ORANGES

18.	The standards and requirements for canned oranges are as follows:

Styles and standards for styles

(1)	(a)	Canned oranges shall be presented as one of the following styles:

(i)	Whole orange segments.

(ii)	Broken orange segments.

(iii)	Orange slices.

(iv)	Orange pieces.

(b)	(i)	Whole orange segments shall consist of peeled orange segments which are practically intact and which retain their original shape:  Provided that the segments may be slightly split.

(ii)	Broken orange segments shall consist of portion of segments which retain at least 50 percent of their original apparent size, or which are large enough to remain on a sieve having 12 mm square openings formed by a wire of 2 mm diameter.

(iii)	Orange slices shall consist of peeled oranges cut transversally into slices.

(iv)	Orange pieces shall consist of portions of segments that are large enough to remain on a sieve having 8 mm square openings formed by a wire of 2 mm diameter

Size grouping in the case of whole orange segments

(2)	Canned whole orange segments shall be grouped according to size in the following manner:

(a)	Large:	20 or less whole orange segments per 100 gram of drained fruit.

(b)	Medium:	21 to 35 whole orange segments per 100 gram of drained fruit.

(c)	Small:	36 or more whole orange segments per 100 gram of drained fruit

# Quality Standards

(3)	(a)	There shall be five grades of canned oranges, namely Fancy Grade, Choice Grade, Standard Grade, Substandard Grade and Manufacturers Grade.

(b)	The quality standards for canned oranges are as follows:

| Quality factor | Fancy Grade | Choice Grade | Standard Grade | Substandard Grade | Manufacturers Grade |
| --- | --- | --- | --- | --- | --- |
| (a) Microbiological spoilage (maximum per production group) | 0,25% | 0,25% | 0,25% | 0,25% | 0,25% |
| (b) Fill of containers shall comply to at least one of the following minima: (i) Fruit quantity | Packed to capacity | As for Fancy Grade | As for Fancy Grade | As for Fancy Grade | As for Fancy Grade |
| (ii) Drained mass in grams in the case of: (aa) Orange segments | 425 | 425 | 425 | 425 | 425 |
| (bb) Orange pieces and slices | 453 | 453 | 453 | 453 | 453 |

| Quality factor | Fancy Grade | Choice Grade | Standard Grade | Substandard Grade | Manufacturers Grade |
| --- | --- | --- | --- | --- | --- |
| (c) Fruit ingredient | Clean, sound and fully grown oranges with similar cultivar characteristics | As for Fancy Grade | As for Fancy Grade | As for Fancy Grade | Clean, sound and fully grown oranges |
| (d) Preparation | (i) Shall be free from defects (ii) Shall be properly peeled and free from residual peel | As for Fancy Grade | (i), (ii), (iii), (iv), (v) and (vi) as for Fancy Grade | As for Standard Grade | Shall be reasonably free from defects, residual peel, fibre strands, developed seeds, segment membranes (where applicable) and off- |
|  | (iii) Shall be free from fibre strands originating from albedo or core |  |  |  | flavours |
|  | (iv) Shall be free from developed seeds |  |  |  |  |
|  | (v) The segment membranes shall have been removed: Provided that the segment membranes may still be attached in the case of -- |  |  |  |  |

| Quality factor | Fancy Grade | Choice Grade | Standard Grade | Substandard Grade | Manufacturers Grade |
| --- | --- | --- | --- | --- | --- |
|  | (aa) orange slices; and (bb) enzymatically peeled oranges;and the membranes shall in both cases be reasonably tender |  |  |  |  |
|  | (vi) Shall be free from off-flavours due to overprocessing, flavours and odours foreign to the product and from excessive rag |  |  |  |  |
|  | (vii) Shall, in the case of orange slices, be free from end-slices |  | * |  |  |
| (e) Uniformity of size in any one container in the case of whole orange segments | The mass of the largest unit shall not be more than twice the mass of the smallest unit | As for Fancy Grade | As for Fancy Grade | * | * |

| Quality factor | Fancy Grade | Choice Grade | Standard Grade | Substandard Grade | Manufacturers Grade |
| --- | --- | --- | --- | --- | --- |
| (f) Maximum thickness in the case of orange slices | 10 mm | 10 mm | 10 mm | * | * |
| (g) Shape in the case of: (i) Whole orange segments | Shall consist of whole segments of a normal shape: Provided that a segment shall be deemed to be whole if it has retained at least 75 percent of its original shape | As for Fancy Grade | As for Fancy Grade | The greater proportion of the units shall be recognisable as whole units | * |
| (ii) Orange slices | Shall consist of whole units of a normal shape | As for Fancy Grade | As for Fancy Grade | The greater proportion of the units shall be recognisable as whole units | * |
| (h) Colour of units | Shall be of a rich yellow to orange colour, typical of the cultivar concerned | As for Fancy Grade | As for Fancy Grade | Shall be of a reasonably good colour | * |
| (i) Texture | Shall be reasonably firm, characteristic of the cultivar concerned and reasonably free from dry cells or fibrous portions which | As for Fancy Grade | As for Fancy Grade | * | * |

| Quality factor | Fancy Grade | Choice Grade | Standard Grade | Substandard Grade | Manufacturers Grade |
| --- | --- | --- | --- | --- | --- |
|  | affect the appearance and edibility of the product |  |  |  |  |
| (j) Foreign matter | None | None | None | None | None |
| (k) Extraneous vegetable matter | Practically free | Fairly free | Reasonably free | Reasonably free | Reasonably free |

*	Denotes no specification

Permissible deviations (by mass per container)

(4)	The maximum permissible deviations by mass per container, unless otherwise specified, for canned oranges are as follows:

| Quality factor | Fancy Grade | Choice Grade | Standard Grade |
| --- | --- | --- | --- |
| (a) Shape in the case of: (i) Whole segments (ii) Orange slices | 30% of the drained mass 8% of the drained mass | 40% of the drained mass 10% of the drained mass | 60% of the drained mass 20% of the drained mass |
| (b) Occurrence of broken segments and pieces in the case of whole orange segments | 3% | 5% | 7% |

| Quality factor | Fancy Grade | Choice Grade | Standard Grade |
| --- | --- | --- | --- |
| (c) Occurrence of pieces in the case of broken orange segments | 8% | 10% | 15% |
| (d) Occurrence in all styles of: (i) Seeds (per sample average) (ii) Fibre stands (aggregate length based on sample average) (iii) Segment membranes, if it has been removed (based on sample average) | 1 unit per 100 g 40 mm per 100 g 5 cm² per 100 g | 1 unit per 100 g 50 mm per 100 g 7 cm² per 100 g | 2 units per 100 g 75 mm per 100 g 10 cm² per 100 g |

CITRUS SEGMENTS (GRAPEFRUIT AND ORANGE SEGMENTS EXCLUDED)

19.	The standards and requirements for canned citrus segments (grapefruit and orange segments excluded) shall be as follows:

Style

(1)	Canned citrus segments (grapefruit and orange segments excluded) shall be presented as whole citrus segments which are practically intact and which retain their original shape:  Provided that the segments may be slightly split.

# Quality standards

(2)	(a)	There shall be five grades for canned citrus segments (grapefruit and orange segments excluded), namely Fancy Grade, Choice Grade, Standard Grade, Substandard Grade and Manufacturers Grade.

(b)	The quality standards for canned citrus segments (grapefruit and orange segments excluded) are as follows:

| Quality factor | Fancy Grade | Choice Grade | Standard Grade | Substandard Grade | Manufacturers Grade |
| --- | --- | --- | --- | --- | --- |
| (a) Microbiological spoilage (maximum per production group) | 0,25% | 0,25% | 0,25% | 0,25% | 0,25% |
| (b) Fill of containers shall comply to at least one of the following minima: (i) Fruit quantity | Packed to capacity | As for Fancy Grade | As for Fancy Grade | As for Fancy Grade | As for Fancy Grade |
| (ii) Drained mass in grams | 425 | 425 | 425 | 425 | 425 |
| (iii) Ingoing mass in grams: (aa) Per container (bb) Average per container | 538 552 | 538 552 | 538 552 | 538 552 | 538 552 |

| Quality factor | Fancy Grade | Choice Grade | Standard Grade | Substandard Grade | Manufacturers Grade |
| --- | --- | --- | --- | --- | --- |
| (c) Fruit ingredient | Clean, sound and fully grown citrus fruit (grapefruit and orange segments excluded) with similar cultivar characteristics | As for Fancy Grade | As for Fancy Grade | As for Fancy Grade | Sound and fully grown citrus fruit (grapefruit and orange segments excluded) |
| (d) Preparation | (i) Shall be free from defects (ii) Shall be properly peeled and free from residual peel | (i), (ii), (iii), (v) and (vi) as for Fancy Grade (iv) Shall be fairly free from developed seeds | (i), (ii), (iii), (v) and (vi) as for Fancy Grade (iv) Shall be reasonably free from developed seeds | As for Standard Grade | Shall be reasonably free from defects, residual peel, fibre strands, developed seeds, segment membranes (where applicable) and off-flavours |
|  | (iii) Shall be free from fibre strands originating from albedo or core |  |  |  |  |
|  | (iv) Shall be free from developed seeds |  |  |  |  |
|  | (v) The segments membranes shall have been removed: Provided that the segment membranes may still be attached |  |  |  |  |

| Quality factor | Fancy Grade | Choice Grade | Standard Grade | Substandard Grade | Manufacturers Grade |
| --- | --- | --- | --- | --- | --- |
|  | in the case of enzymatically peeled citrus segments (grapefruit and orange segments excluded), in which the membranes shall be reasonably tender |  |  |  |  |
|  | (vi) Shall be free from off-flavours due to over-processing, flavours and odours foreign to the product and from excessive rag |  |  |  |  |
| (e) Uniformity of size in any one container in the case of whole citrus (grapefruit and orange segments excluded) | The mass of the largest unit shall not be more than twice the mass of the smallest unit | As for Fancy Grade | As for Fancy Grade | * | * |
| (f) Shape | Shall consist of whole segments of a normal shape: Provided that a segment shall be deemed to be whole if it has re- | As for Fancy Grade | As for Fancy Grade | * | * |

| Quality factor | Fancy Grade | Choice Grade | Standard Grade | Substandard Grade | Manufacturers Grade |
| --- | --- | --- | --- | --- | --- |
|  | tained at least 75 percent of its original shape |  |  |  |  |
| (g) Colour of units | Shall be of a colour typical of the cultivar concerned | As for Fancy Grade | As for Fancy Grade | * | * |
| (h) Foreign matter | None | None | None | None | None |
| (i) Extraneous vegetable matter | Practically free | Fairly free | Reasonably free | Reasonably free | Reasonably free |

*	Denotes no specification

Permissible deviations (by mass per container)

(3)	The maximum permissible deviations for canned citrus segments (grapefruit and orange segments excluded) are as follows:

| ### Quality factor | Fancy Grade | Choice Grade | Standard Grade |
| --- | --- | --- | --- |
| (a) Shape | 20% of the drained mass | 40% of the drained mass | 60% of the drained mass |

CITRUS TWO FRUIT

Quality standards

20.	(1)	(a)	There shall be five grades of canned citrus two fruit namely Fancy Grade, Choice Grade, Standard Grade, Sub-

standard Grade and Manufacturers Grade.

(b)	The standards and requirements for canned citrus two fruit are as follows:

| Quality factor | Fancy Grade | Choice Grade | Standard Grade | Substandard Grade | Manufacturers Grade |
| --- | --- | --- | --- | --- | --- |
| (a) Microbiological spoilage (maximum per production group) | 0,25% | 0,25% | 0,25% | 0,25% | 0,25% |
| (b) Fill of containers shall comply to at least one of the following minima: |  |  |  |  |  |
| (i) Fruit quantity | Packed to capacity | As for Fancy Grade | As for Fancy Grade | As for Fancy Grade | As for Fancy Grade |
| (ii) Drained mass in gram | 425 | 425 | 425 | 425 | 425 |
| (iii) Ingoing mass in grams: (aa) Per container (bb) Average per container | 538 552 | 538 552 | 538 552 | 538 552 | 538 552 |

| Quality factor | Fancy Grade | Choice Grade | Standard Grade | Substandard Grade | Manufacturers Grade |
| --- | --- | --- | --- | --- | --- |
| (c) Maximum count | 69 | * | * | * | * |
| (d) Allowable shape | Segments | Segments | Segments | Segments | * |
| (e) Fruit ingredient | Clean, sound and fully grown citrus fruit with similar cultivar characteristics | As for Fancy Grade | As for Fancy Grade | As for Fancy Grade | Clean, sound and fully grown citrus fruit |
| (f) Preparation | (i) Shall be free from defects (ii) Shall be properly peeled and free from residual peel (iii) Shall be free from fibre strands originating from albedo or core | As for Fancy Grade | As for Fancy Grade | As for Fancy Grade | Shall be reasonably free from defects, residual peel, fibre strands, developed seeds, segment membranes (where applicable) and off-flavours |
|  | (iv) Shall be free from developed seeds |  |  |  |  |
|  | (v) The segment membranes shall have been removed: Provided that the segment membranes may still be attached |  |  |  |  |

| Quality factor | Fancy Grade | Choice Grade | Standard Grade | Substandard Grade | Manufacturers Grade |
| --- | --- | --- | --- | --- | --- |
|  | in the case of enzymatically peeled citrus fruit in which case the membranes shall be reasonably tender |  |  |  |  |
|  | (vi) Shall be free from off-flavours due to overprocessing, flavours and odours foreign to the product and from excessive rag |  |  |  |  |
| (g) Uniformity of size in any one container | Segments shall be fairly uniform in size | Segments shall be reasonably uniform in size | As for Choice Grade | * | * |
| (h) Shape | Shall consist of whole segments of a normal shape: Provided that a segment shall be deemed to be whole if it has retained at least 75% of its original shape | As for Fancy Grade | As for Fancy Grade | The greater proportion of the units shall be recognisable as whole units | * |
| (i) Minimum quantity | Each constituent shall not be less than 40% of the total drained mass | As for Fancy Grade | Each constituent shall not be less than 35% of the total drained mass | As for Standard Grade | * |

| Quality factor | Fancy Grade | Choice Grade | Standard Grade | Substandard Grade | Manufacturers Grade |
| --- | --- | --- | --- | --- | --- |
| (j) Colour | Units of each constituent in a container shall be of a good and uniform colour throughout | Units of each constituent in a container shall be of a fairly good colour throughout | Units of each constituent in a container shall be of a reasonably good colour throughout | As for Standard Grade | * |
| (k) Foreign matter | None | None | None | None | None |
| (l) Extraneous vegetable matter | Practically free from | Fairly free | Reasonably free | Reasonably free | Reasonably free |

*	Denotes no specification

Permissible deviations (by mass per container)

(2)	The maximum permissible deviations for canned citrus two fruits are as follows:

| ## Quality factor | Fancy Grade | Choice Grade | Standard Grade |
| --- | --- | --- | --- |
| (a) Shape | 20% of the drained mass | 40% of the drained mass | 60% of the drained mass |

PAWPAWS

21.	The standards and requirements for canned pawpaws are as follows:

Styles and standards for styles

(1)	(a)	Canned pawpaws shall be presented as one of the following styles:

(i)	Pawpaw slices.

(ii)	Pawpaw chunks.

(iii)	Pawpaw diced.

(b)	(i)	Pawpaw slices shall consist of peeled and deseeded pawpaws cut into wedge shaped units.

(ii)	Pawpaw chunks shall consist of peeled and deseeded pawpaws cut into irregular shaped and sized units.

(iii)	Pawpaw diced shall consist of peeled and deseeded pawpaws cut into approximately cube-shaped units.

Quality Standards

(2)	(a)	There shall be four grades of canned pawpaws, namely Fancy Grade, Choice Grade, Substandard Grade and Manufacturers Grade.

(b)	The quality standards for canned pawpaws are as follows:

| Quality factor | Fancy Grade | Choice Grade | Substandard Grade | Manufacturers Grade |
| --- | --- | --- | --- | --- |
| (a) Microbiological spoilage (maximum per production group) | 0,25% | 0,25% | 0,25% | 0,25% |
| (b) Fill of containers shall comply to at least one of the following minima: |  |  |  |  |
| (i) Fruit quantity | Packed to capacity | As for Fancy Grade | As for Fancy Grade | As for Fancy Grade |
| (ii) Drained mass in grams | 481 | 481 | 481 | 481 |
| (c) Fruit ingredient | Sound, well coloured and fully grown pawpaws which have similar cultivar characteristics | As for Fancy Grade | As for Fancy Grade | Sound and fully grown pawpaws |
| (d) Preparation | (i) Shall be free from defects | As for Fancy Grade | As for Fancy Grade | Shall be reasonably free from defects, developed seeds and residual peel |

| Quality factor | Fancy Grade | Choice Grade | Substandard Grade | Manufacturers Grade |
| --- | --- | --- | --- | --- |
|  | (ii) Shall be properly peeled and free from residual peel |  |  |  |
|  | (iii) Shall be free from seeds |  |  |  |
|  | (iv) Dices shall be cleanly cut and more or less cube-shaped |  |  |  |
| (e) Mushiness | Units shall not be mushy | As for Fancy Grade | The greater proportion of the units shall be recognisable as whole units | * |
| (f) Colour of units | Shall be good and uniform | Shall be fairly good and reasonably uniform | As for Choice Grade | * |
| (g) Foreign matter | None | None | None | None |
| (h) Extraneous vegetable matter | Practically free | Fairly free | Reasonably free | Reasonably free |

*	Denotes no specification

Permissible deviations (by number per container)

(3)	The maximum permissible deviations for canned pawpaws are as follows:

| Quality factor | Fancy Grade | Choice Grade |
| --- | --- | --- |
| (a) Mushiness in the case of: (i) A2½-containers | 3 | 5 |
| (ii) Containers larger or smaller than A2 ½-containers | In the same proportion as 3 to 853 ml | In the same proportion as 5 to 853 ml |

FREESTONE PEACHES

22.	The standards and requirements for canned freestone peaches are as follows:

Definitions

(1)	Where used with regard to canned freestone peaches --

(a)	"crushed/broken" means a unit, in the case of whole peaches, peach halves and peach quarters, which has been --

(i)	crushed to the extent that it has lost its normal shape (not due to ripeness);  or

(ii)	severed into definite parts:  Provided that all portions that collectively equal the size of a full size unit are considered one unit in applying the permissible deviations.

(b)	"freestone peach" means peach where the pit separates readily from the flesh;

(c)	"pit/stone material" means, in the case of all styles other than whole peaches, pits and pieces of pit (including pit extensions) that are dark, hard and sharp and which are at least 1,5 mm in any dimension:  Provided that it shall not be considered as a defect when whole peach pits or peach kernels are used as seasoning ingredients.

Styles and standards for styles

(2)	(a)	Canned freestone peaches shall be presented as one of the following styles:

(i)	Whole peaches.

(ii)	Peach halves.

(iii)	Peach quarters.

(iv)	Peach slices.

(v)	Peach diced.

(vi)	Peach pieces, mixed pieces or irregular pieces.

(b)	(i)	Whole peaches shall consist of whole, peeled and unpitted peaches.

(ii)	Peach halves shall consist of peeled and pitted peaches cut into two approximately equal parts.

(iii)	Peach quarters shall consist of peeled and pitted peaches cut into four approximately equal parts.

(iv)	Peach slices shall consist of peeled and pitted peaches cut into wedge shaped units.

(v)	Peach diced shall consist of peeled and pitted peaches cut into approximately cube-shaped units.

(vi)	Peach pieces, mixed pieces or irregular pieces shall consist of peeled and pitted peaches cut into irregular shaped and sized units.

Quality Standards

(3)	(a)	There shall be five grades of canned freestone peaches, namely Fancy Grade, Choice Grade, Standard Grade, Substandard Grade and Manufacturers Grade.

(b)	The quality standards for canned freestone peaches are as follows:

| Quality factor | Fancy Grade | Choice Grade | Standard Grade | Substandard Grade | Manufacturers Grade |
| --- | --- | --- | --- | --- | --- |
| (a) Microbiological spoilage (maximum per production group) | 0,25% | 0,25% | 0,25% | 0,25% | 0,25% |
| (b) Fill of containers shall comply to at least one of the following minima: |  |  |  |  |  |
| (i) Fruit quantity | Packed to capacity | As for Fancy Grade | As for Fancy Grade | As for Fancy Grade | As for Fancy Grade |
| (ii) Drained mass in grams | 453 | 453 | 453 | 453 | 453 |
| (iii) Ingoing mass in grams: |  |  |  |  |  |

| Quality factor | Fancy Grade | Choice Grade | Standard Grade | Substandard Grade | Manufacturers Grade |
| --- | --- | --- | --- | --- | --- |
| (aa) Per container in the case of: - Lye peeled halves and quarters - Hand peeled halves and quarters | 481 510 | 481 510 | 481 510 | 481 510 | 481 510 |
| - Lye and hand peeled slices | 481 | 481 | 481 | 481 | 481 |
| - Lye and hand peeled irregular pieces | - | - | 481 | 481 | 481 |
| (bb) Average per container in the case of: - Lye peeled halves and quarters - Hand peeled halves and quarters | 510 538 | 510 538 | 510 538 | 510 538 | 510 538 |

| Quality factor | Fancy Grade | Choice Grade | Standard Grade | Substandard Grade | Manufacturers Grade |
| --- | --- | --- | --- | --- | --- |
| - Lye and hand peeled slices - Lye and hand peeled irregular pieces | 510 - | 510 - | 510 510 | 510 510 | 510 510 |
| (c) Maximum count in the case of: (i) Halves (ii) Quarters | 14 30 | 22 50 | 32 55 | 32 55 | * * |
| (d) Minimum count | 6: Provided that halves in containers with a capacity of between 400 ml and 500 ml shall either have a minimum count of 5 or be marked according to subitem (5) | * | * | * | * |
| (e) Fruit ingredients | Sound freestone peaches with similar cultivar characteristics | As for Fancy Grade | As for Fancy Grade | As for Fancy Grade | Sound freestone peaches |

| Quality factor | Fancy Grade | Choice Grade | Standard Grade | Substandard Grade | Manufacturers Grade |
| --- | --- | --- | --- | --- | --- |
| (f) Preparation | (i) Shall be free from dark discoloured stone cavities and off-suture cuts: Provided that off-suture cuts 10 mm or less (at its widest point) which do not detract markedly from the appearance of the fruit shall not be regarded as deviations | (i) and (ii) as for Fancy Grade | As for Choice Grade | As for Choice Grade except that the units may be trimmed: Provided that the greater proportion of the units in a container shall be the style described on the label | Shall be reasonably free from defects, residual peel, peach stones and parts of peach stones |
|  | (ii) Shall be neatly and cleanly peeled and shall be free from residual peel |  |  |  |  |
|  | (iii) May be trimmed if the units do not thereby lose their normal shape: Provided that peach halves may not be trimmed | (iii)(aa) May be trimmed: Provided that the units do not there by lose their normal shape |  |  |  |

| Quality factor | Fancy Grade | Choice Grade | Standard Grade | Substandard Grade | Manufacturers Grade |
| --- | --- | --- | --- | --- | --- |
|  |  | (bb) in the case of halves trimming on the outer edges is allowed: Provided the trimming is restricted to the contour of the unit |  |  |  |
|  | (iv) Shall in the case of slices -- | (iv) and (v) as for Fancy Grade |  |  |  |
|  | (aa) be free from slivers and slabs; and |  |  |  |  |
|  | (bb) be free from butt, calyx and suture slices which detract from the overall appearance of the contents |  |  |  |  |

| Quality factor | Fancy Grade | Choice Grade | Standard Grade | Substandard Grade | Manufacturers Grade |
| --- | --- | --- | --- | --- | --- |
|  | (v) Shall be free from peach stones or parts of peach stones |  |  |  |  |
| (g) Minimum length in the case of slices (mea- sured from tip to tip) | 45 mm | 40 mm | 25 mm | 25 mm | * |
| (h) Uniformity of size in any one container in the case of: (i) Halves and quarters | The mass of the x largest units in a container shall be not more than y times the mass of the x smallest units where x = 4 and y = 1½ | As for Fancy Grade except that x = 5 and y = 1½ | (i) Halves: As for Fancy Grade except that x = 10 and y = 2 (ii) Quarters: As for Fancy Grade except that x = 20 and y = 2 | * | * |
| (ii) Slices | Shall be reasonably uniform in thickness: Provided that slices which are not uniform in size are allowable if the container is marked accordingly | As for Fancy Grade | As for Fancy Grade | * | * |

| Quality factor | Fancy Grade | Choice Grade | Standard Grade | Substandard Grade | Manufacturers Grade |
| --- | --- | --- | --- | --- | --- |
| - Shape of halves | Shall be normal | As for Fancy Grade | As for Fancy Grade | * | * |
| (j) Colour of units | Shall be of good colour | Shall be of a fairly good colour | Shall be of a reasonably good colour | As for Standard Grade | * |
| (k) Uniformity of colour in any one container | Units in a container shall be uniform in colour and deviations from the general colour shall not be conspicuous | Shall be fairly uniform in colour throughout | Shall be reasonably uniform in colour throughout | As for Standard Grade | * |
| (l) Crushing (halves and quarters) | Units shall not be crushed or broken or damaged by crushing | As for Fancy Grade | As for Fancy Grade | As for Fancy Grade | * |
| (m) Foreign matter | None | None | None | None | None |
| (n) Extraneous vegetable matter | Practically free | Fairly free | Reasonably free | Reasonably free | Reasonably free |

*	Denotes no specification

-	Denotes not applicable

Permissible deviations (by number per container)

(4)	The maximum permissible deviations for canned freestone peaches are as follows:

| Quality factor | Fancy Grade | Choice Grade | Standard Grade | Substandard Grade |
| --- | --- | --- | --- | --- |
| (a) Uniformity of colour in any one container | 25% | 0% | 0% | 0% |
| (b) Crushing in the case of: (i) Halves (ii) Quarters | 14,3% 6,7% | 13,6% 6% | 15,6% 9,1% | 15,6% 9,1% |
| (c) Minimum length of slices | 20%: Provided that the general appearance is not affected detrimentally | 20% | 20% | 20% |
| (d) Preparation in the case of: (i) Halves (ii) Quarters (iii) Slices (by mass) | 14,3% 6,7% 10% | 18,2% 8% 15% | 31,3% 18,2% 20% | 31,3% 18,2% 20% |

| Quality factor | Fancy Grade | Choice Grade | Standard Grade | Substandard Grade |
| --- | --- | --- | --- | --- |
| (iv) Irregular pieces | - | - | 20% | 30% |
| (e) Deviations in (b)(i) and (d)(i) collectively: Provided such defects are individually within the limits specified | 14,5% | 18,2% | 31,5% | 31,5% |
| (f) Deviations in (b)(ii) and (d)(ii) collectively: Provided such defects are individually within the limits specified | 6,7% | 8% | 18,2% | 18,2% |

*	Denotes no specification

-	Denotes not applicable

Additional marking requirements

(5)	If the requirement for the minimum number of peach halves as prescribed in subitem (3)(d) are not complied with, the count shall be stated on the label.

YELLOW CLINGSTONE PEACHES

23.	The standards and requirements for canned yellow clingstone peaches are as follows:

# Definitions

- Where used with regard to yellow clingstone peaches --

- “clingstone peach” means a peach which have the pit adhered to the flesh.

- “crushed/broken” means a unit, in the case of whole peaches, peach slices, peach halves and peach quarters, which has been --

- crushed to the extent that it has lost its normal shape (not due to ripeness);  or

- severed into definite parts:  Provided that all portions that collectively equal the size of a full size unit are considered one unit in applying the permissible deviations.

- “pit/stone material” means, in the case of all styles other than whole peaches, pits and pieces of pit (including pit extensions) that are dark, hard and sharp and which are at least 1,5 mm in any dimension:  Provided that it shall not be considered as a defect when whole peach pits or peach kernels are used as seasoning ingredients.

Styles and standards for styles

(2)	(a)	Canned  yellow  clingstone  peaches  shall  be  presented  as  one  of  the

following styles:

- Whole peaches.

- Peach halves.

- Peach quarters.

- Peach slices.

- Peach diced.

- Peach pieces, mixed pieces or irregular pieces.

(b)	(i)	Whole  peaches   shall   consist   of  whole,   peeled   and   unpitted

peaches.

(ii)	Peach halves shall consist of peeled and pitted peaches cut into two approximately equal parts.

- Peach quarters shall consist of peeled and pitted peaches cut into four approximately equal parts.

- Peach slices shall consist of peeled and pitted peaches cut into wedge shaped units.

- Peach diced shall consist of peeled and pitted peaches cut into approximately cube-shaped units.

- Peach pieces, mixed pieces or irregular pieces shall consist of peeled and pitted peaches cut into irregular shaped and sized units.

## Quality Standards

(3)	(a)	There shall be five grades of canned yellow clingstone peaches, namely Fancy Grade, Choice Grade, Standard Grade, Substandard Grade and Manufacturers Grade.

(b)	The quality standards for canned yellow clingstone peaches are as follows:

| Quality factor | Fancy Grade | Choice Grade | Standard Grade | Substandard Grade | Manufacturers Grade |
| --- | --- | --- | --- | --- | --- |
| - Microbiological spoilage (maximum per production group) | 0,25% | 0,25% | 0,25% | 0,25% | 0,25% |
| (b) Fill of containers shall comply to at least one of the following minima: |  |  |  |  |  |
| (i) Fruit quantity | Packed to capacity | As for Fancy Grade | As for Fancy Grade | As for Fancy Grade | As for Fancy Grade |
| (ii) Drained mass in grams in the case of: |  |  |  |  |  |
| (aa) Wholes | - | 453 | 453 | 453 | 453 |
| (bb) Halves, quarters, slices and diced | 467 | 467 | 467 | 467 | 467 |
| (cc) Irregular pieces | - | - | 467 | 467 | 467 |

| Quality factor | Fancy Grade | Choice Grade | Standard Grade | Substandard Grade | Manufacturers Grade |
| --- | --- | --- | --- | --- | --- |
| (iii) Ingoing mass in grams: |  |  |  |  |  |
| (aa) Per container in the case of: |  |  |  |  |  |
| - Wholes | - | 481 | 481 | 481 | 481 |
| - Halves, quar- ters, slices, and diced | 467 | 467 | 467 | 467 | 467 |
| - Irregular pieces | - | - | 467 | 467 | 467 |
| (bb) Average per container in the case of: |  |  |  |  |  |
| - Wholes | - | 510 | 510 | 510 | 510 |
| - Halve, quarters, slices and diced | 496 | 496 | 496 | 496 | 496 |
| - Irregular pieces | - | - | 496 | 496 | 496 |

| Quality factor | Fancy Grade | Choice Grade | Standard Grade | Substandard Grade | Manufacturers Grade |
| --- | --- | --- | --- | --- | --- |
| (c) Maximum count in the case of: |  |  |  |  |  |
| (i) Wholes | - | 20 | 20 | 20 | * |
| (ii) Halves | 22 | 25 | 36 | 36 | * |
| (iii) Quarters | 30 | 50 | 55 | 55 | * |
| (d) Minimum count | 6: Provided that halves in containers with a ca-pacity of between 400 ml and 500 ml shall either have a minimum count of 5 or be marked according to subitem (5): Provided further that in the case of M1 containers a minimum count of 4 is allowable | * | * | * | * |
| (e) Fruit ingredients | Sound yellow cling-stone peaches with similar cultivar characteristics | As for Fancy Grade | As for Fancy Grade | As for Fancy Grade | Sound yellow cling-stone peaches |
| (f) Preparation | (i) Shall be free from dark discoloured | (i) and (ii) as for Fancy Grade | As for Choice Grade | As for Choice Grade except that the units | Shall be reasonably free from defects, residual |

| Quality factor | Fancy Grade | Choice Grade | Standard Grade | Substandard Grade | Manufacturers Grade |
| --- | --- | --- | --- | --- | --- |
|  | stone cavities and off-suture cuts: Provided that off-suture cuts 10 mm or less (at its widest point) which do not detract markedly from the appearance of the fruit shall not be regarded as deviations |  |  | may be trimmed: Pro-vided that the greater proportion of the units in a container shall be recognisable as of the style described on the label | peel, peach stones and parts of peach stones |
|  | (ii) Shall be properly peeled and free from residual peel |  |  |  |  |
|  | (iii) Units may not be trimmed | (iii) (aa) Units may be trimmed: Pro-vided that they do not thereby lose their nor-mal shape. |  |  |  |

| Quality factor | Fancy Grade | Choice Grade | Standard Grade | Substandard Grade | Manufacturers Grade |
| --- | --- | --- | --- | --- | --- |
|  |  | (bb) In the case of halves trimming on the outer edges is allowed: Provided the trimming is restricted to the contour of the unit |  |  |  |
|  |  | (cc) Slight “scoop- ing” of the stone cavities are acceptable: Provided that halves are still fleshy |  |  |  |
|  | (iv) Shall be free from peach stones or parts of peach stones and dark, hard pit extensions | (iv) As for Fancy Grade: Provided that the stones shall not be removed in the case of wholes |  |  |  |
|  | (v) All slices shall be cleanly cut | (v), (vi) and (vii) as for Fancy Grade |  |  |  |

| Quality factor | Fancy Grade | Choice Grade | Standard Grade | Substandard Grade | Manufacturers Grade |
| --- | --- | --- | --- | --- | --- |
|  | (vi) Shall in the case of slices-- |  |  |  |  |
|  | (aa) be free from sli- vers and slabs; and |  |  |  |  |
|  | (bb) be free from butt, calyx and suture slices |  |  |  |  |
|  | which detract from the overall appearance of the contents |  |  |  |  |
|  | (vii) Diced shall be clearly cut and shall be free from chips which in the case of diced -- |  |  |  |  |
|  | (aa) smaller than 13 mm, shall pass through a sieve with 5 mm meshes; and |  |  |  |  |

| Quality factor | Fancy Grade | Choice Grade | Standard Grade | Substandard Grade | Manufacturers Grade |
| --- | --- | --- | --- | --- | --- |
|  | (bb) of 13 mm and more, shall pass through a sieve with 6 mm meshes |  |  |  |  |
| (g) Minimum length of slices (measured from tip to tip) | 45 mm | 40 mm | 25 mm | 25 mm | * |
| (h) Uniformity of size in any one container in the case of: |  |  |  |  |  |
| (i) Wholes | * | The mass of the x largest units in a container shall be not more than y times the mass of the x smallest units where x=4 and y=1½. | As for Choice Grade except that x=6 and y=2 | * | * |
| (ii) Halves | The mass of the x largest units in a container shall be not more than y times larger than the mass of the x smallest units where – | As for Fancy Grade | As for Fancy Grade except that -- | * | * |

| Quality factor | Fancy Grade | Choice Grade | Standard Grade | Substandard Grade | Manufacturers Grade |
| --- | --- | --- | --- | --- | --- |
|  | (aa) x=2 and y=1½ for a count of 7 or under |  | (aa) x=2 and y=2 for a count of 7 or under |  |  |
|  | (bb) x=3 and y=1½ for a count of 8 or over |  | (bb) x=4 and y=2 for a count of 8 or over |  |  |
| (iii) Quarters | That mass of the x largest units in a container shall not be more than y times larger than | As for Fancy Grade except that -- | As for Fancy Grade except that -- | * | * |
|  | the mass of the x smallest units, where -- |  |  |  |  |
|  | (aa) x=2 and y=1½ for a count of 10 or less | (aa) x=3 and y=1½ for a count of 10 or less | (aa) x=2 and y=2 for a count of 10 or less |  |  |
|  | (bb) x=4 and y=1½ for a count of 11 to 20 | (bb) x=5 and y=1½ for a count of 11 to 20 | (bb) x=4 and y=2 for a count of 11 to 20 |  |  |

| Quality factor | Fancy Grade | Choice Grade | Standard Grade | Substandard Grade | Manufacturers Grade |
| --- | --- | --- | --- | --- | --- |
|  |  | (cc) x=6 and y=1½ for counts of 21 and over | (cc) x=6 and y=2 for a count of 21 to 40 |  |  |
|  |  |  | (dd) x=10 and y=2 for a count of 41 and over |  |  |
| (iv) Slices | Shall be reasonably uniform and the mass of the x largest units in a container shall not be more than y times larger | As for Fancy Grade except that -- | As for Choice Grade | * | * |
|  | than the mass of the x smallest units, where -- |  |  |  |  |
|  | (aa) x=3 and y=2 for a count of 10 or under | (aa) x=3 and y=3 for a count of 10 or under |  |  |  |
|  | (bb) x=5 and y=2 for a count of 11 to 20 | (bb) x=5 and y=3 for a count of 11 to 20 |  |  |  |

| Quality factor | Fancy Grade | Choice Grade | Standard Grade | Substandard Grade | Manufacturers Grade |
| --- | --- | --- | --- | --- | --- |
|  | (cc) x=6 and y=2 for a count of 21 and over: Provided that partial slices shall be ignored | (cc) x=6 and y=3 for counts of 21 and over |  |  |  |
| (i) Shape of halves | Shall be normal | As for Fancy Grade | As for Fancy Grade | * | * |
| (j) Colour of units in the case of: |  |  |  |  |  |
| (i) Wholes | - | Shall be of a fairly good colour throughout | Shall be of a reasonably good colour throughout | As for Standard Grade | * |
| (ii) Halves | Shall be of a good colour and shall show no serious red colouration around the stone | Shall be of a fairly good colour | Shall be of a reasonably good colour | As for Standard Grade | * |
| (iii) Quarters and slices | Shall be of a good colour | Shall be of a fairly good colour | Shall be of a reasonably good colour | As for Standard Grade | * |
| (iv) Diced | Shall be of a good colour | Shall be of a fairly good colour | Shall be of a reasonably good colour | As for Standard Grade | * |

| Quality factor | Fancy Grade | Choice Grade | Standard Grade | Substandard Grade | Manufacturers Grade |
| --- | --- | --- | --- | --- | --- |
| (v) Irregular pieces | - | - | Shall be of reasonably good colour throughout | As for Standard Grade | * |
| (k) Uniformity of colour in any one container in the case of: |  |  |  |  |  |
| (i) Halves, quarters, slices and diced | Units in container shall be uniform in colour and deviations from the | Shall be of a fairly uniform colour throughout | Shall be of a reasonably uniform colour throughout | As for Standard Grade | * |
|  | general colour shall not be conspicuous |  |  |  |  |
| (ii) Irregular pieces | - | - | Shall be of a reasonably uniform colour throughout | As for Standard Grade | * |
| (l) Crushing | Units shall not be crushed or broken or damaged by crushing (disintegrated) | As for Fancy Grade | As for Fancy Grade | As for Fancy Grade | * |
| (m) Foreign matter | None | None | None | None | None |
| (n) Extraneous vegetable matter | Practically free | Fairly free | Reasonably free | Reasonably free | Reasonably free |

*	Denotes no specification

-	Denotes not applicable

## Permissible deviations (by number per container)

(4)	The maximum permissible deviations for canned yellow clingstone peaches are as follows:

| Quality factor | Fancy Grade | Choice Grade | Standard Grade | Substandard Grade |
| --- | --- | --- | --- | --- |
| (a) Uniformity of colour in any one container in the case of halves, quarters, slices and dices | 25% | 0% | 0% | 0% |
| (b) Crushing | 6,7% | 9,1% | 16,7% | 16,7% |
| (c) Shape of halves | 0% | 0% | 20% | *: Provided that the style of units in a container shall be recognizable as the style on the label |
| (d) Minimum length of slices | 20%: Provided that the general appearance is not affected detrimentally | 20% | 20% | *: Provided that the style of units in a container shall be recognizable as the style on the label |
| (e) Preparation in the case of: |  |  |  |  |
| (i) Wholes | - | 30% | 40% | 40% |

| Quality factor | Fancy Grade | Choice Grade | Standard Grade | Substandard Grade |
| --- | --- | --- | --- | --- |
| (ii) Halves | 15%: Provided that if 15% represents less than one unit per container a full unit shall be allowed | 20% | 25% | 25% |
| (iii) Quarters | 15% | 20% | 25% | 25% |
| (iv) (aa) Slices (by mass) | 10% | 15% | 20% | 20% |
| (bb) Butt-end, calyx ends, slivers and slabs | - | - | 10% | *: Provided that the style of units in a container shall be recognizable as the style on the label |
| (v) Diced (by mass) | 10% | 15% | 20% | 20% |
| (vi) Irregular pieces | - | - | 30% | 30% |
| (f) Size of dices (by mass) | 20% | 30% | 40% | 40% |
| (g) Off-suture cuts | 15% | 25% | 40% | * |
| (h) (i) Deviations in para- graphs (b), (c), (e)(ii) and (g) collectively: Provided such defects are individually within | 15%: Provided if 15% represents less than one unit per container a full unit shall be allowed | 20% | 40% | 40% |

| Quality factor | Fancy Grade | Choice Grade | Standard Grade | Substandard Grade |
| --- | --- | --- | --- | --- |
| the limits specified above (halves) |  |  |  |  |
| (ii) Deviations in (b), (c) and (e)(iii), (iv), (v) collectively: Provided such defects are individually within the limits specified above (quarters, slices, dices) | 15% | 20% | 25% | 25% |

*	Denotes no specification

-	Denotes not applicable

## Additional marking requirements

(5)	If the requirements for the minimum number of peach halves as prescribed in subitem (3)(d) are not compiled with, the count shall be stated on the label.

PEACH PULP AND PEACH PIE FRUIT (SOLID PACK PEACH)

24.	The standards and requirements for canned peach pulp and peach pie fruit (solid pack peach) are as follows:

Styles and standards for styles

(1)	(a)	Canned peach pie fruit (solid pack peach) shall be presented as unscreened, which shall consist of either peach halves, peach quarters, peach segments or peach slices.

(b)	Canned peach pulp shall be presented as screened, which shall consist of peaches comminuted into granular particles.

Quality Standards

(2)	(a)	There shall be three grades for canned peach pulp and peach pie fruit (solid pack peaches), namely Fancy Grade, Choice Grade and Manufacturers Grade.

(b)	The quality standards for canned peach pulp and peach pie fruit (solid pack peaches) are as follows:

| Quality factor | Fancy Grade | Choice Grade | Manufacturers Grade |
| --- | --- | --- | --- |
| (a) Microbiological spoilage (maximum per production group) | 0,25% | 0,25% | 0,25% |
| (b) Fill of containers | Shall be packed to capacity and shall contain not more than the minimum amount of water required for processing | As for Fancy Grade | As for Fancy Grade |
| (c) Fruit ingredient | Sound, well-coloured peaches | As for Fancy Grade | Reasonably sound peaches |
| (d) Preparation | Shall be free from defects | As for Fancy Grade | Shall be reasonably free from defects |
| (e) Colour | Shall be well-coloured | Shall be fairly well-coloured | Shall be reasonably well-coloured |
| (f) Blemishes | Shall be free from blemishes | Shall be fairly free from blemishes | Shall be reasonably free from blemishes |
| (g) Foreign matter | None | None | None |
| (h) Extraneous vegetable matter | Practically free | Fairly free | Reasonably free |