Standards regarding Food Safety and Food Hygiene of Regulated Agricultural Products of Plant Origin destined for Export.

- as stipulated under 4(3)(a)(ii) of the Agricultural Products Standards Act 119 of 1990, and promulgated in notice R707 of 13 May 2005.

## FBO Type:  Processing Facilities

Revision 2 of October 2008

Implementation date:  01 August 2008

CHECKLIST

Scope: Permanent facilities involved with the semi-processing/processing

of all products of plant origin destined for export.

Not applicable is not justified

| R/P | Item | Control Point | Level | #### Compliance Yes / No / N/a | Comments |
| --- | --- | --- | --- | --- | --- |

| 1. |  |  |  | ##### TRACEABILITY |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 |  |  |  | Is product traceable at the processing facility? |  |  |  |  |  |  |  |  |  |  |
| R/P |  | 1.1.1 |  | # Is product that is received identified? | # MAJOR |  |  |  |  |  |  |  |  |  |
| R/P |  | 1.1.2 |  | # Is the source of the product received (i.e. the “sender” of the product) identified? | # MAJOR |  |  |  |  |  |  |  |  |  |
| R/P |  | 1.1.3 |  | # Is product traceable within the processing facility? | # MAJOR |  |  |  |  |  |  |  |  |  |
| 1.2 |  |  |  | Are products that leave the facility trackable to the next entity? |  |  |  |  |  |  |  |  |  |  |
| R/P |  | 1.2.1 |  | # Is the processing facility identified as the “sender” of the product? | # MAJOR |  |  |  |  |  |  |  |  |  |
| R/P |  | 1.2.2 |  | # Is the product that leaves the processing facility identified? | # MAJOR |  |  |  |  |  |  |  |  |  |
| R/P |  | 1.2.3 |  | # Is the next receiver in the supply chain identified? | # MAJOR |  |  |  |  |  |  |  |  |  |
| 2. |  |  |  | # HACCP PRINCIPLES |  |  |  |  |  |  |  |  |  |  |
| R |  |  | 2.1 | # Is a document available to indicate that a hazard analysis of the process in the processing plant/factory has been done taking into consideration biological, chemical and physical hazards? |  | MAJOR |  |  |  |  |  |  |  |  |
| R |  |  | 2.2 | # Based on the documented hazard analysis, have the Critical Control Points (CCPs) been correctly identified? |  | MAJOR |  |  |  |  |  |  |  |  |
| R |  |  | 2.3 | # Are critical limits set and established for the identified CCPs? |  | MAJOR |  |  |  |  |  |  |  |  |
| R |  |  | 2.4 | # Are procedures in place to monitor these critical limits? |  | MAJOR |  |  |  |  |  |  |  |  |
| R |  |  | 2.5 | # If monitoring indicates that a critical limit for an identified CCP has been exceeded, what corrective procedures have been established? |  | MAJOR |  |  |  |  |  |  |  |  |
| R |  |  | 2.6 | # Are records available for all identified CCPs? |  | MAJOR |  |  |  |  |  |  |  |  |
| R |  |  | 2.7 | # What verification procedures have been established to determine if the control measures for the identified CCP’s are effective for the purpose? |  | MAJOR |  |  |  |  |  |  |  |  |
| # 3. |  |  |  | # LOCATION OF THE FACILITY |  |  |  |  |  |  |  |  |  |  |
| P |  | 3.1 |  | Is the facility not located in an area that poses a threat to food safety? |  |  | MINOR |  |  |  |  |  |  |  |
| P |  | 3.2 |  | Is the intake and dispatch area protected from the environmental factors (rain, dust etc.) and bird droppings? |  |  | MINOR |  |  |  |  |  |  |  |
| # 4. |  |  |  | #### WATER SUPPLY |  |  |  |  |  |  |  |  |  |  |
| R |  | 4.1 |  | Is the water used for sanitation and processing purposes (as an ingredient or for the purpose of washing, hydro-cooling, ice-cooling) potable, i.e. drinking quality? |  |  |  | CRITICAL |  |  |  |  |  |  |
| R |  | 4.2 |  | Where recycled water is used for process purposes, is it treated and maintained in a condition that will not constitute a risk to produce? |  |  |  | CRITICAL |  |  |  |  |  |  |
| R/P |  | 4.3 |  | Does non-potable water for fire control, steam production, refrigeration have a separate water system, where applicable? |  |  |  | MAJOR |  |  |  |  |  |  |
| 5. |  |  |  | # DRAINAGE AND WASTE DISPOSAL |  |  |  |  |  |  |  |  |  |  |
| R/P |  | 5.1 |  | Are all waste disposal containers and areas specifically identified, cleaned and maintained? |  |  |  | MINOR |  |  |  |  |  |  |
| P |  | 5.2 |  | Is a drainage system provided for efficient disposal of wastewater to prevent the contamination of fresh produce or the potable water source? |  |  |  | MINOR |  |  |  |  |  |  |
| P |  | 5.3 |  | Are the grounds in the immediate vicinity of the pack house free of litter, waste and improperly stored garbage, and is grass on the packhouse premises kept short? |  |  |  | MINOR |  |  |  |  |  |  |
| 6. |  |  |  | # PERSONAL HYGIENE |  |  |  |  |  |  |  |  |  |  |
| R |  | 6.1 |  | Does the packhouse have documented hygiene instructions? |  |  |  | CRITICAL |  |  |  |  |  |  |
| P |  | 6.2 |  | Do workers implement the hygiene instructions for produce handling? |  |  |  | # MAJOR |  |  |  |  |  |  |
| R |  | 6.3 |  | Are workers trained in basic hygiene? |  |  |  | # MAJOR |  |  |  |  |  |  |
| P |  | 6.4 |  | Do workers have access to first-aid facilities? |  |  |  | # MAJOR |  |  |  |  |  |  |
| P |  | 6.5 |  | Do all workers and visitors wear suitable clean protective clothing? |  |  |  | MINOR |  |  |  |  |  |  |
| P |  | 6.6 |  | Are adequate, suitable and conveniently located sanitary facilities available? |  |  |  | MAJOR |  |  |  |  |  |  |
| P |  | 6.7 |  | Are sanitary facilities maintained? |  |  |  | # MAJOR |  |  |  |  |  |  |
| P |  | 6.8 |  | Are visible signboards directing workers to wash hands present, instructing workers to wash hands before product is handled? |  |  |  | MINOR |  |  |  |  |  |  |
| 7. |  |  |  | STORAGE (CLEANING CHEMICALS, POST HARVEST CHEMICALS) |  |  |  |  |  |  |  |  |  |  |
| P |  | 7.1 |  | # Is the storage facility for the storage of cleaning/post harvest chemicals suitable for the purpose and secure? |  |  |  | MINOR |  |  |  |  |  |  |
| P |  | 7.2 |  | Are all chemicals properly labelled? |  |  |  | # MINOR |  |  |  |  |  |  |
| 8. |  |  |  | FOOD CONTROL AND MONITORING EQUIPMENT |  |  |  |  |  |  |  |  |  |  |
| R |  | 8.1 |  | Where applicable, are process controls (such as temperature, humidity etc.) regarding this equipment (e.g. cold stores, retorts etc.) controlled and monitored in the interest of preventing product contamination? |  |  |  | # CRITICAL |  |  |  |  |  |  |
| R |  | 8.2 |  | Where applicable, is all monitoring equipment calibrated? |  |  |  | # MAJOR |  |  |  |  |  |  |
| 9. |  |  |  | FACILITY: MAINTENANCE, CLEANING, PEST CONTROL and PREVENTION & CONTROL OF PRODUCT CONTAMINATION. |  |  |  |  |  |  |  |  |  |  |
| 9.1 |  |  |  | Maintenance and cleaning |  |  |  |  |  |  |  |  |  |  |
| P | 9.1.1 |  |  | Are packhouse, storage facilities, pack line equipment/machinery surfaces (sizers, conveyors, drop boards, grading tables and wash tanks) coming into contact with produce well maintained and free from flaking plaster and metal shards, to ensure that it is cleanable and avoid injury to product? |  | # MAJOR |  |  |  |  |  |  |  |  |
| P | 9.1.2 |  |  | Are cleaning chemicals used according to the manufacturer’s instructions? |  | MAJOR |  |  |  |  |  |  |  |  |
| R | 9.1.3 |  |  | Is there a cleaning schedule in place that defines the different areas and equipment to be cleaned, the frequency, responsibility and verification procedures? |  | MINOR |  |  |  |  |  |  |  |  |
| R | 9.1.4 |  |  | Is designated cleaning equipment used? |  | MINOR |  |  |  |  |  |  |  |  |
| P | 9.1.5 |  |  | Is packaging material clean and stored in a clean and hygienic state? |  | # MINOR |  |  |  |  |  |  |  |  |
| R/P | 9.1.6 |  |  | Are there written procedures or checks for monitoring of glass and hard plastic, and action should a breakage occur? |  | MINOR |  |  |  |  |  |  |  |  |
| 9.2 |  |  |  | Pest Control Systems |  |  |  |  |  |  |  |  |  |  |
| R/P | 9.2.1 |  |  | Is a formalized, ongoing and well-maintained pest control programme implemented? |  | MAJOR |  |  |  |  |  |  |  |  |
| P | 9.2.2 |  |  | If FBO manages own pest control programme as required in 10.2.1, are the chemicals stored in a secure, place according to label instructions? |  | MINOR |  |  |  |  |  |  |  |  |
| 9.3 |  |  |  | Physical Contamination |  |  |  |  |  |  |  |  |  |  |
| R/P | 9.3.1 |  |  | Are there preventative measures in place to prevent foreign matter contaminating produce? |  | MAJOR |  |  |  |  |  |  |  |  |
| R/P | 9.3.2 |  |  | Are control measures in place to eliminate foreign matter from produce to an allowable level, where a tolerance is specified? |  | CRITICAL |  |  |  |  |  |  |  |  |

##### AUDIT SUMMARY

|  | Control points / Checklist | Number N/A justified | Results of this Audit |  |
| --- | --- | --- | --- | --- |
|  |  |  | No of control points applicable | No of control points n/a |
| TOTAL NO OF CRITICAL CONTROL POINTS | 5 | 2 |  |  |
| TOTAL NO OF MAJOR CONTROL POINTS | 24 | 7 |  |  |
| TOTAL NO OF MINOR CONTROL POINTS | 14 | 1 |  |  |
| TOTAL CONTROL POINTS | 43 |  |  |  |

General Comments …………………………………………………………………………………………………………………………………………………………….

……… ……………………………………………………………………………………………………………………………………………………

………………………………………………………………………………………………………………………………………………………………

………………………………………………………………………………………………………………………………………………………………

………………………………………………………………………………………………………………………………………………………………

………………………………………………………………………………………………………………………………………………………………

Auditor Signature …………………………….		FBO Representative Signature……………………FBO Code ………………………

Name……………………(………………………)		Name ……………………………………………………………………………………

Date ………./………………./………..		                                        Date ………./……………………./………………..