Standards regarding Food Safety and Food Hygiene of Regulated Agricultural Products of Plant Origin destined for Export.

- as stipulated under 4(3)(a)(ii) of the Agricultural Products Standards Act 119 of 1990, and promulgated in notice R707 of 13 May 2005.

## FBO Type:  Dry Storage facilities

Revision:  October 2008

Implementation date: 01 October 2007

COMPLIANCE CRITERIA, NOTES AND MANAGEMENT AIDS

### Scope: Dry storage facilities involved in the handling, storage and distribution of products of plant origin, which are intended for export.  These include commercial storage facilities and exclude facilities only storing own product.

NOTES AND MANAGEMENT AIDS

| #1 | REGISTRATION PPECB would confirm the registration status of a Food Business Operator and establishment(s) before arranging an audit. |  |
| --- | --- | --- |
| #1.1 | #### Have relevant establishments been registered with the Department of Agriculture and allocated FBO Codes? | The Food Hygiene and Food Safety Standard issued under the APS Act [Agricultural Product Standards Act, 1990 (Act No. 119 of 1990)] specifies that a Food Business Operator (FBO) must register establishments that handle products of plant origin that are destined for export. Each registered establishment is allocated an FBO Code. The FBO is responsible for keeping registrations and details associated with the FBO Code up to date. Details relating to FBO registrations are available on the Department of Agriculture website, www.nda.agric.za in “Regulatory and Other Services: Quality Assurance and Food Safety” section. |
| #1.2 | #### Have food products produced by an establishment been registered for and identified with that establishment? | A FBO must identify food products produced by each establishment. The food products are associated with the relevant FBO Code. The FBO is responsible for keeping details relating to food products up-to-date. |
| #2 | CERTIFICATION PPECB would confirm the certification status of a Food Business Operator and establishment(s) before arranging an audit. |  |
| #2.1 | #### Has a FBO (establishment) been certified as complying with a standard such as EurepGAP, HACCP or BRC? | PPECB establishes the priority and frequency of audits based on the risk profile of a FBO (establishment). A FBO (establishment) that is certified as complying with a recognized standard would usually be considered to be a relatively lower risk than a FBO (establishment) that has no certification. |

NOTES AND MANAGEMENT AIDS

| #3. | #### RECORD KEEPING FOR FOOD SAFETY PURPOSES |
| --- | --- |
|  | Records provide evidence of diligent operations and compliance with requirements and, in the event of something going wrong, may be vital to investigations and consequential actions such as product withdrawals. The FBO (establishment) is responsible for implementing a suitable record keeping system. As illustrations of statement of requirements: - The Food Hygiene and Food Safety Standard issued under the APS Act [Agricultural Product Standards Act, 1990 (Act No. 119 of 1990)] requires a primary production food business operator to: - Establish a record system for each field, orchard or greenhouse to provide a permanent record of activities undertaken at those locations; - Establish a visual identification or reference system for each field, orchard or greenhouse; - Keep records or documentation containing the following relevant information for at least 2 years: - Suppliers of agricultural inputs and lot numbers of agricultural inputs; - Irrigation practices; - The correct and applicable use of plant protection products and fertilizers; - The proper treatment of manure or biosolids or organic fertilizers; - Cleaning and waste disposal programmes; and - The results of analyses carried out on samples taken of products or of other analysis made with regard to: - Plant protection products; - Water quality; - Soil contamination. - Quality Standards and Food Hygiene and Food Safety Standards issued under the APS Act [Agricultural Product Standards Act, 1990 (Act No. 119 of 1990)] specify that, for traceability purposes, food business operators should keep adequate records which allow them to identify the suppliers of ingredients and food products used in their operations, as well as food businesses supplied with products. Requirements relating to records are specified in the food safety checklist questions and compliance criteria, and highlighted by a “R”. Requirements relating to physical evidence are specified in the food safety checklist questions and compliance criteria, and highlighted by “P” - Records may not always be available at the point of operations during the audit e.g. were record sheets to be processed in the administration or information services department. Nevertheless, operational management and staff should know where all relevant records would be held, and the auditor should have access to the records requested during an audit. - Historic records would usually not be examined during a food safety audit. It would be good practice for the FBO (establishment) and operational management and staff to review the findings of a food safety audit with regard to the quality and availability of records in each aspect of operation assessed by answering under each section of this checklist whether the records requested during the audit were available, and were there any specific considerations to be noted or actions required. A sample summary and review sheet is provided on the next page, for off farm produce handling facilities. |

NOTES AND MANAGEMENT AIDS

| #### Records summary/review worksheet –Dry storage facilities (This document is not audited.) |  |  |
| --- | --- | --- |
| Section 1 | TRACEABILITY - Are records requested during the audit available? - Are any specific considerations to be noted or actions required? |  |
| Section 2 | HACCP PRINCIPLES - Are records requested during the audit available? - Are any specific considerations to be noted or actions required? |  |
| Section 3 | LOCATION OF STORAGE FACILITY - Are records requested during the audit available? - Are any specific considerations to be noted or actions required? |  |
| Section 4 | WATER SUPPLY - Are records requested during the audit available? - Are any specific considerations to be noted or actions required? |  |
| Section 5 | DRAINAGE AND WASTE DISPOSAL - Are records requested during the audit available? - Are any specific considerations to be noted or actions required? |  |
| Section 6 | PERSONAL HYGIENE - Are records requested during the audit available? - Are any specific considerations to be noted or actions required? |  |
| Section 7 | STORAGE (CLEANING CHEMICALS) - Are records requested during the audit available? - Are any specific considerations to be noted or actions required? |  |
| Section 8 | FOOD CONTROL AND MONITORING EQUIPMENT - Are records requested during the audit available? - Are any specific considerations to be noted or actions required? |  |
| Section 9 | FACILITY MAINTENANCE, CLEANING AND PEST CONTROL - Are records requested during the audit available? - Are any specific considerations to be noted or actions required? |  |
| Section 10 | MANAGING PRODUCT WITHDRAWALS - Are records requested during the audit available? - Are any specific considerations to be noted or actions required? |  |

NOTES AND MANAGEMENT AIDS

| #4. | MANAGING WITHDRAWALS |
| --- | --- |
|  | Quality Standards and Food Hygiene and Food Safety Standards issued under the APS Act hold producers, packhouses and exporters responsible for establishing traceability of products at all stages of production, packing, handling and distribution and – with relevant FBOs - for withdrawing products that present a serious risk to the health of consumers. The Codex Alimentarius “Principles And Guidelines For The Exchange Of Information In Food Safety Emergency Situations” (CAC/GL 19-1995, Rev. 1-2004) provides guidance for use by national governments and regional economic integration organizations for the exchange of information in food safety emergency situations. These Principles and Guidelines apply to situations where the competent authorities in either the importing and/or exporting countries become aware of a food safety emergency situation, and communication of the information and risks surrounding the emergency situation must be undertaken. Each country should designate a primary official contact point for food safety emergency situations, which can act as the national focal point for information exchange in such situations The following food safety authorities may play a key role in the event of any significant food safety alert, incident and action involving produce exported or destined for export. Department of Health; National Department of Agriculture. Directorate: Food Control; Directorate: Food Safety and Quality Assurance. Mr AWJ Pretorius; Ms J Rathebe. Pretoria; Agricultural Place, Room LAFF:09. Tel: (012) 312 0185; 20 Beatrix Street. Fax: (012) 312 3180; PRETORIA. E-mail: pretoa@@health.gov.za; Tel: (012) 319 7000. Fax: (012) 319 6764. E-mail: DFSQA@nda.agric.za. |

| R/P | Item | Control Point | Level | Compliance Criteria / Guidance Notes |
| --- | --- | --- | --- | --- |

| 1. |  |  |  | ##### TRACEABILITY |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 |  |  |  | Is product traceable at the storage facility? |  |  |  |  |  |  |  |
| R/P |  | 1.1.1 |  | # Is product that is received at the storage facility identified? | # MAJOR |  |  | Record the products received at the storage facility. - Product identity e.g. product type and cultivar - Containers, where applicable e.g. bulk bins, cartons etc. - Logistic units where applicable e.g. pallets or containers - Quantity (mass or amount of units, where applicable) - Date received (Time – where applicable) |  |  | Different farms, pack houses and contracted transporters may deliver products to the storage facility in an unplanned and unpredicted sequence. A logistic unit is a unit of products that is created for the purposes of efficient storage and transportation. |
| R/P |  | 1.1.2 |  | # Is the source of the product received (i.e. the “sender” of the product) identified? | # MAJOR |  |  | Record the source (sender) of the product received. - FBO Code of the sending facility e.g. farm, pack house etc. - Sub-farm unit e.g. orchards / storage rooms etc. (where applicable) - Name and address - Details of the transport that delivered the product (where applicable). |  |  | Full traceability requires that the source (sender) of the product be recorded, whether or not the source (sender) is a registered FBO. |
| R/P |  | 1.1.3 |  | # Is product traceable within the storage facility? | # MAJOR |  |  | Record all movements of products in the storage facility - What product - From-location / time (as appropriate) - To-location / time (as appropriate) |  |  |  |
| R/P |  | 1.1.4 |  | # Are new products that are created or reconstituted by the storage facility identified and traceable? | # MAJOR |  |  | Record each new product and constitution of product that is created at the facility - Consolidations (e.g. of part pallets into full pallets, loading pallets into containers, transport etc. - Sampling - Finished product identification (including batch number if appropriate) - Timing |  |  | Record changes in constitution - the newly constituted item would be the trackable item from the storage facility forward. The store identity (FBO Code) may be used as the source of new or consolidated products created. Sampling procedures should ensure that samples are traceable to source products and that sampled material is correctly managed and is traceable. |
| 1.2 |  |  |  | Are products that leave storage facility trackable to the next entity? |  |  |  |  |  |  |  |
| R/P |  | 1.2.1 |  | # Is the storage facility identified as the “sender” of the product? |  |  | # MAJOR |  | Record the storage facility identity (FBO Code) as the sender of the product. |  |  |
| R/P |  | 1.2.2 |  | # Is the product that leaves the storage facility identified? |  |  | # MAJOR |  | Record the identities of products that leave the storage facility - Product identity and description in accordance with marking requirements - Batch identification - Containers or constitution (e.g. pallets) - Item identities (if appropriate) (e.g. pallet numbers) |  | Full traceability requires that all products be identified and traceable, including those that is diverted from the export chain to the local market and industrial processing, or for waste. |
| R/P |  | 1.2.3 |  | # Is the next receiver in the supply chain identified? |  |  | # MAJOR |  | Record the identity of the next receiving entity - FBO Code of a consolidation store, processor, transporter, port terminal etc. - Name and address of receiving entity. - Transporter FBO Code and vehicle details. |  | Full traceability, and the ability to support recalls or withdrawals, would require all recipients to be recorded, whether or not registered FBOs. |
| 2. |  |  |  | # HACCP PRINCIPLES |  |  |  |  |  |  |  |
| R | 2.1 |  |  | # Is a document available to indicate that a hazard analysis of the process in the store has been done taking into consideration biological, chemical and physical hazards, and if a hazard was identified that is not addressed by the measures indicated in this checklist, were appropriate control points implemented to control the hazard? |  | MAJOR |  |  | Refer to HACCP Guidelines / Industry where available. |  |  |
| # 3. |  |  |  | # LOCATION OF STORAGE FACILITY |  |  |  |  |  |  |  |
| P |  |  | 3.1 | Is the storage facility not located in an area that poses a threat to food safety? |  | MINOR |  |  | The store should not be located in areas where: -contamination of food may occur by means of environmentally polluted and industrial areas. -it will be prone to infestations of pests. |  |  |
| P |  |  | 3.2 | Is the intake and dispatch area protected from the environmental factors (rain, dust etc.) and bird droppings? |  | MINOR |  |  | Preferably no loading must take place during unfavourable conditions. In cases where loading does take place, the product must be suitably protected. |  |  |
| # 4. |  |  |  | #### WATER SUPPLY |  |  |  |  |  |  |  |
| R |  |  | 4.1 | Is the water used for sanitation purposes (washing) potable, i.e. drinking quality? |  | MAJOR |  |  | The water has been declared suitable by the competent authorities and/or within the last 12 months a water analysis has been carried out at the point of entry into the facility. Annual testing: Microbial Limits must not exceed the following parameters: E Coli – not detectable (0) Thermotolerant faecal Coliforming bacteria – 10/100ml |  |  |
| 5. |  |  |  | # DRAINAGE AND WASTE DISPOSAL |  |  |  |  |  |  |  |
| R/P |  |  | 5.1 | Are all waste disposal containers and areas specifically identified, cleaned and maintained? |  | MINOR |  |  | Waste disposal containers/areas must be identified and managed to prevent cross-contamination of product. The area must be routinely cleaned and maintained (removal of waste), and cleaned according to the cleaning schedule. |  |  |
| P |  |  | 5.2 | Is a drainage system provided for efficient disposal of wastewater to prevent the contamination of fresh produce or the potable water source? |  | MINOR |  |  | Floors must be designed with i.e. slopes, drainage channels and kept free and clear to ensure drainage. If the structure does not allow adequate drainage, the cleaning procedure must detail removal of excess water. |  |  |
| P |  |  | 5.3 | Are the grounds in the immediate vicinity of the store free of litter, waste and improperly stored garbage, and is grass on the premises kept short? |  | MINOR |  |  | Long grass will harbour pest activity. |  |  |
| 6. |  |  |  | # PERSONAL HYGIENE |  |  |  |  |  |  |  |
| R |  |  | 6.1 | Does the storage facility have documented hygiene instructions? |  | MAJOR |  |  | The instructions must at least include: - the need for hand cleaning; - the covering of skin cuts; - limitation on smoking, eating and drinking to certain areas; - notification of any relevant infections or conditions; - wearing of jewellery - the use of suitable protective clothing. It should also be included in a hygiene instruction that staff must report illnesses to management. |  |  |
| P |  |  | 6.2 | Do workers implement the hygiene instructions? |  | # MAJOR |  |  | # There is evidence that the workers are complying with hygiene instructions (Physical observation). |  |  |
| R |  |  | 6.3 | Are workers trained in basic hygiene? |  | # MINOR |  |  | # Evidence of basic hygiene training (content of hygiene instruction) must be available. Is this training repeated at a set interval (e.g. yearly/seasonally) to refresh personnel about the company’s hygiene requirements. There is evidence (i.e.: signed attendance registration, external certificates) that the workers have received verbal and documented understandable instructions in the relevant aspects of produce handling hygiene including: personal cleanliness i.e. hand washing, wearing of jewellery and fingernail length and cleaning etc. clothing cleanliness, personal behaviour, i.e. no smoking, spitting, eating, chewing, perfumes, etc.). |  |  |
| P |  |  | 6.4 | Do workers have access to first-aid facilities? |  | # MINOR |  |  | # Maintained first aid kits and recommendations must be available and accessible in the vicinity of the work. (These kits must as a minimum include bandages, plasters and disinfectant). |  |  |
| P |  |  | 6.5 | Do all workers and visitors wear suitable clean protective clothing? |  | MINOR |  |  | Protective clothing: Should cover personal clothing, without external pockets and buttons. |  |  |
| P |  |  | 6.6 | Are adequate, suitable and conveniently located sanitary facilities available? |  | MAJOR |  |  | Adequate: Approximately 15 persons to one toilet facility. If number of people exceeds 10, then separate men’s and women’s toilets should be provided. Running water must be available for washing of hands. Hand washing facilities must be available at the entrance or vicinity to the packhouse. |  |  |
| P |  |  | 6.7 | Are sanitary facilities maintained? |  | # MAJOR |  |  | # Toilet paper, liquid disposable soap, and where disposable paper towels are used, a bin for paper waste. |  |  |
| P |  |  | 6.8 | Are visible signboards directing workers to wash hands present, instructing workers to wash hands before product is handled? |  | MINOR |  |  | The hygiene instructions are visibly displayed: provided by way of clear signs (pictures) or in the predominant language(s) of the workforce. |  |  |
| 7. |  |  |  | STORAGE (CLEANING CHEMICALS) |  |  |  |  |  |  |  |
| P |  |  | 7.1 | # Is the storage facility for the storage of cleaning chemicals suitable for the purpose and secure? |  | MINOR |  |  | # The storage facility must be secure and access only allowed to trained personnel. # The facility must allow adequate ventilation, be well lit, be able to retain spillage, and be of sound construction. |  |  |
| P |  |  | 7.2 | Are all chemicals properly labelled? |  | # MINOR |  |  | # All the chemicals kept in the store must be kept in the original containers and packs, in case of breakage only, the new package must contain all the information of the original label. |  |  |
| 8. |  |  |  | ##### STORAGE FACILITY - MANAGEMENT PROCESSES |  |  |  |  |  |  |  |
| P |  | 8.1 |  | Is the intake and dispatch area protected from the environmental factors (rain, dust etc.) and bird droppings? |  |  | MAJOR |  |  |  |  |
| P/R |  | 8.2 |  | Before loading, are procedures in place to, check the condition of transport / insulated or covered transport? |  |  | MINOR |  |  | Physical check will be done by the auditor. The record can be in the form of a checklist or a note on the receiving/intake document. |  |
| P/R |  | 8.3 |  | Does the storage of product take into account compatibility and storage protocols? (e.g. tainting, contamination) |  |  | MAJOR |  |  | Is a storage protocol/procedure or table available indicating to personnel what products can safely be stored together and at what preferred temperature and humidity regimes. Reference can be made to the PPECB blue book that can be found at: http://www.ppecb.com/NR/rdonlyres/1EACDDC4-278E-4D39-824B-5585A2F23152/448/Section22006Directory13Mb.pdf The auditor must do a physical check to verify that product are stored to prevent tainting, e.g. products such as avocado’s, according to the protocol presented. The facility’s use must be restricted to food only. |  |
| R/P |  | 8.4 |  | Are damaged/contaminated goods isolated and disposed of in accordance with a procedure? |  |  | MAJOR |  |  | If product is contaminated / damaged, does the procedure prescribe how the product will be handled? E.g. isolating from distribution and making sure that product is not consumed by humans. Is rejected product segregated and marked accordingly. |  |
| 9. |  |  |  | MAINTENANCE, CLEANING AND PEST CONTROL |  |  |  |  |  |  |  |
| 9.1 |  |  |  | Maintenance and cleaning |  |  |  |  |  |  |  |
| P |  | 9.1.1 |  | Are all surfaces free of flaking paint, corrosion, rust and other materials (e.g. tape, wood etc?) |  |  | MAJOR |  |  | Flaking paint, equipment chipping etc. will constitute a physical hazard to product. They should be made of smooth, non-absorbent materials, and inert to the food, to detergents and disinfectants under normal operating conditions. It must also be constructed of material facilitating effective cleaning, such as plastic, stainless steel, cement, except wood. All floor surfaces must be in good condition with no standing water. |  |
| P |  | 9.1.2 |  | Are cleaning chemicals used according to the manufacturer’s instructions? |  |  | MINOR |  |  | Documentary evidence exists authorizing (i.e. specific label mention or technical data sheet) use for the food industry of Cleaning Agents, Lubricants etc. which may come into contact with produce. |  |
| R |  | 9.1.3 |  | Is there a cleaning schedule in place that defines the different areas and equipment to be cleaned, the frequency, responsibility and verification procedures? |  |  | MAJOR |  |  | This must include sanitary, change rooms, floors, walls, surrounding and waste disposal areas. |  |
| P |  | 9.1.4 |  | Are all lights in the packing/processing facilities covered with protective covers and is all other glass protected to prevent contamination in case of breakage? |  |  | MAJOR |  |  | Light bulbs and fixtures suspended above produce or material used for produce handling are of a safety type or are protected / shielded so as to prevent contamination of food in case of breakage. |  |
| R/P |  | 9.1.5 |  | Are there written procedures or checks for monitoring of glass and hard plastic, and action should a breakage occur? |  |  | MINOR |  |  | Written procedures and checks exist for handling glass or clear hard plastic breakages in produce handling, preparation and storage areas. Monitoring of all glass to check whether it is still intact, as well as corrective action if a breakage does occur. |  |
| R/P |  | 9.1.6 |  | Is a procedure in place to separate and replace dirty or broken pallets? |  |  | MINOR |  |  | The procedure must stipulate segregation that must be exercised to prevent unintended use. |  |
| 9.2 |  |  |  | Pest Control |  |  |  |  |  |  |  |
| R/P |  | 9.2.1 |  | Is there a procedure for pest control monitoring and treatments? |  |  | MAJOR |  |  | Establishments and surrounding areas must be regularly examined for evidence of infestation, and any treatments or activity recorded. External contractor or own trained staff. Frequency to be identified in the procedure. Service records must be available. Monitoring traps must be numbered indicated on a site map. Pest control traps must be maintained and cleaned on a regular basis. Pest control traps must be secured. |  |
| P |  | 9.2.2 |  | Are openings to doorways, vents and drains, covered to prevent the ingress of pests, birds and other animals? |  |  | MINOR |  |  | Facility doors must be kept closed at all times. Pest control measures must be put in place : sieves, air curtains, rodent boxes, light traps etc. |  |

| R/P | Item | Control Point | Level | Compliance Criteria |
| --- | --- | --- | --- | --- |

| # 10. |  | # MANAGING PRODUCT WITHDRAWALS (NB: This section relates to new requirements and is not yet included in audit scores.) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| R | 10.1 | Has a specific person on site been given the responsibility to manage food safety investigations and withdrawals, and is that person’s contact details available? | Audited/ Not scored | Each FBO (establishment) should have a written procedure for managing a food safety incident or alert, withdrawing products, and communicating in this regard with authorities and other FBOs / companies who may be involved. The procedure would identify responsibilities and authority, and prescribe controls and reporting relating to the incident and/or affected products. | Guidelines in this regard will be provided in the Traceability Standard Operating Procedure. |
| R | 10.2 | Are procedures in place to respond to and manage a trace information request? | Audited/ Not scored | Each FBO (establishment) should have a written procedure for responding to requests for traceability and food safety information related to a food safety incident or alert. Such a procedure would allow for access to historic/archived information, verification of the accuracy of information, and authorization for release of information. | Guidelines in this regard will be provided in the Traceability Standard Operating Procedure. |