| 1 | Product Receiving | Truck standing temperature / time before off-loading | Q |  | Handling procedure |
| --- | --- | --- | --- | --- | --- |
|  |  | Product standing – temperature / time after off-loading | Q |  | Handling procedure |
|  |  | Withoulding periods have been complied to | S (Chemical) | CCP | -Procedure for checking spraying records to allowable MRL. -Verification via laboratory test results. |
| 2 | Drenching | Chemical – overdosing | S (Chemical) | CCP | Procedure for controlling: -Sign-off sheets for chemical measurement. This might take the form of a batch sign off, and countersigned by authorized person. -Technical Responsiility for determining suggested application quantities. -Standard for topping up depleted drench chemicals.- |
|  |  | Chemical - underdosing | Q |  |  |
|  |  | Changing of dirty drench chemicals | S (Biological) | CCP |  |
| 3 | De-greening | Ethylene / concentration | Q |  | -Handling procedure and equipment to measure temperature. -Measuring equipment should be calibrated and checked for accuracy |
|  |  | Temperature over / under | Q |  |  |
|  |  | Time over / under | Q |  |  |
| 4 | Pre Cooling | Temperature over / under | Q |  | -Procedure for monitoring temperature (pulp and room). -Effective record-keeping. |
|  |  | Time over / under | Q |  |  |
|  |  | Handling of fruit | Q |  |  |
|  |  | Broken pallets / bins segregated | S(Physical) | PRP | -Procedure for minimizing contamination of wood, eg. a control procedure to illiminate chipped pallets/bins. |
|  |  | Light coverings | S(Physical) | PRP | -A control procedure to cover /laminate all lights and glass windows. -A checklist for monitoring glass breakage etc. |
| 5 | Presorting | General water quality | S (Chemical /Biological) | PRP | -Municipal water certificates -Laboratory results ensuring compliance to potable water standards |
|  |  | Poort sorting | Q |  | Inline QC Checks |
| 6 | Washing | Concentration of Chemical(s) too high | S (Chemical) | CCP | Procedure for controlling: -Sign-off sheets for chemical measurement. This might take the form of a batch sign off, and countersigned by authorized person. -Technical Responsiility for determining suggested application quantities. -Standard for topping up of depleted/used chemicals, to ensure that active ingredient is always present in recommended volumes.- |
|  |  | Concentration of Chemical(s) too low | Q |  |  |
| 7 | Rinse/Brushing | Rinse water – concentration of chemical(s) too high | S (Chemical) | CCP | Procedure for controlling: -Sign-off sheets for chemical measurement. This might take the form of a batch sign off, and countersigned by authorized person. -Technical Responsiility for determining suggested application quantities. -Standard for topping up of depleted/used chemicals, to ensure that active ingredient is always present in recommended volumes. -If water is circulated is there a filter that is cleaned and sanitized. |
|  |  | Overbrushing | Q |  | -Procedure for good operational practices should be in place. |
|  |  | Incomplete drying | Q |  |  |
|  |  | General Water Quality | S (Chemical /Biological) | PRP | -Municipal water certificates -Laboratory results ensuring compliance to potable water standards. |
| 8 | Fungacide bath (citrus) | Concentration too high | S (Chemical) | CCP | Procedure for controlling: -Sign-off sheets for chemical measurement. This might take the form of a batch sign off, and countersigned by authorized person. -Technical Responsility for determining suggested application quantities. -Standard for topping up of depleted/used chemicals, to ensure that active ingredient is always present in recommended volumes. |
|  |  | Concentration too low | Q |  |  |
|  |  | General Water Quality | S | PRP | Municipal certificate / Laboratory testing results to potable water standards. |
| 9 | Drying | Time | Q |  | Best Operating Practice (BOP) in place |
|  |  | Temp | Q |  |  |
| 10 | Waxing | Under application | Q |  | BOP Waxing |
|  |  | Over application | Q |  |  |
|  |  | Under dosing chemical(s) | Q |  | Procedure for controlling: -Sign-off sheets for chemical measurement. This might take the form of a batch sign off, and countersigned by authorized person. -Technical Responsility for determining suggested application quantities. |
|  |  | Over dosing chemical(s) | S (Chemical) | CCP |  |
| 11 | Drying | Mechanical damage | Q |  | BOP Drying after waxing |
|  |  | Temp | Q |  |  |
|  |  | Time | Q |  |  |
| 12 | Sorting | Micro & Physical contamination | S (Biological / Chemical) | PRP | Prerequisite Programm(PRP) - Personnel Hygiene and Operating Standards, eg. handwash etc. |
|  |  | Handling | Q |  | -BOP Sorting -Inline QC Check – Sorting Effectiveness |
|  |  | Poor sorting | Q |  |  |
| 13 | Packing | Poor sizing | Q |  | BOP Sorting |
|  |  | Poor colour sorting | Q |  |  |
|  |  | Mechanical damage | Q |  |  |
|  |  | Handling | Q |  |  |
|  |  | Micro and Physical contamination | S (Biological / Chemical) | PRP | -Prerequisite Programm(PRP) - Personnel Hygiene and Operating Standards, eg. handwash etc. -Prerequisite Programme (PRP) – Cleaning practices -Prerequisite Programme (PRP) – Pest control programme |
|  |  | Scale calibration (inaccuracy) | Q | PRP | Yearly calibration of scales, and regular inhouse-checks. Records and certificates must be available. |
| 14 | Marking of Cartons | Incorrect number of ct / pallet | Q |  | BOP Carton Marking |
|  |  | Using wrong securing sheets | Q |  |  |
| 15 | Palletise | Wrong pallets used | Q |  | BOP Palletizing |
|  |  | Poor strapping practice | Q |  |  |
| 16 | Cooling / Container loading | Maintaining Temp (Q) Loading time | Q |  | -BOP Despatch -PPECB Container inspection / Temperature Inspection |
|  |  | Loading correct fruit to consignment order | Q |  |  |
|  |  | Container conditions | S (Biological /Physical) |  |  |

Typical CCP Control Plan – Citrus Packing

| PROCESS STEP | HAZARD TYPE | PREVENTIVE MEASURE | CRITICAL LIMIT / MEASURE | MONITORING |  |  | RESP | CORRECTIVE ACTION | VERIFICATION |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | PROC | FREQ | REC. |  |  | METHOD | FREQ & RESP |
| Product Receiving | Chemical | -Adhere to product application standard -spraying programmes | Importing country MRL | Check withholding periods | Every batch | Spraying records | Packhouse Manager | Isolate and recall the product | Laboratory testing method | Every variety per season Packhouse Manager |
| Drenching | Chemical | Adherence to chemical application standard | Correct mixture/concentration | Check amount of chemical added to bath | Each batch | Batch make-up records | Packhouse Manager | Isolate and recall the product | Batch make up counter-checked by 2nd person | Every bath |
|  | Biological | Cleaning procedure for change of drench water | Amount of bins drenched | Monitor the amount of bins through the bath | Bin count | Water change after bon count has been reached | Packhouse Manager | Isolate and recall the product | Do swabbing of fruit surface | Once per season to set bath operating standard |
| Washing | Chemical | Adherence to chemical application standard | Correct mixture / concentration | Check amount of chemical added to bath | Every bath mixture | Batch make-up records | Packhouse Manager | Isolate and recall the product | Batch make up counter-checked by 2nd person | Every bath |
| Rinse / Brushing | Chemical | Adherence to chemical application standard | Correct mixture / concentration | Check amount of chemical added to tank | Every bath mixture | Batch make-up records | Packhouse Manager | Isolate and recall the product | Batch make up counter-checked by 2nd person | Every bath |
| Fungacide Bath | Chemical | Adherence to chemical application standard | Correct mixture / concentration | Check amount of chemical added to water | Every batch | Batch make-up records | Packhouse Manager | Isolate and recall the product | Batch make up counter-checked by 2nd person | Every Batch |
|  | Biological | Cleaning procedure for change of bath water | Weight of fruit washed | Monitor the amount of fruit through the bath. | Amount of fruit through bath | Batch make-up and top-up records | Packhouse Manager | Isolate and recall the product | Batch make up counter-checked by 2nd person | Every Batch |
| Waxing | Chemical | Adherence to chemical application standard | Correct mixture/concentration | Check amount of chemical added to wax mixture |  | Batch make-up records | Packhouse Manager | Isolate and recall the product | Batch make up counter-checked by 2nd person | Every Batch |

NB:  A Control Point is a CCP if the hazard can not be controlled in a later step in the process, and there is no PRP(Prerequisite Programme) to bring it into control.