| 1 | Product Receiving | Truck standing temperature / time before off-loading | Q |  | Handling procedure |
| --- | --- | --- | --- | --- | --- |
|  |  | Product standing – temperature / time after off-loading | Q |  | Handling procedure |
|  |  | Withoulding periods have been complied to | Chemical | CCP | -Procedure for checking spraying records to allowable MRL. -Verification via laboratory test results. |
| 2 | Long Stem Bath | Non-potable water source | Biological / Chemical | PRP | -Municipal water certificates -Laboratory results ensuring compliance to potable water standards |
|  |  | Chemical contamination-latex neutraliser(Overdosage) | Chemical | CCP | Procedure for controlling: -Sign-off sheets for chemical measurement. This might take the form of a batch sign off, and countersigned by authorized person. -Technical Responsility for determining suggested application quantities. -Standard for topping up of depleted/used chemicals, to ensure that active ingredient is always present in recommended volumes. -Procedure for cleaning/change of bath water to ensure max microbial count. |
|  |  | Dirty bath water-Recontamination of fruit | Biological | CCP |  |
|  |  | Manual removal of stems&placing fruit into bath /latex damage due to wrong handling | Q |  | Handling procedure |
|  |  | Insufficient cleaning of bath | Biological | PRP | Cleaning procedure |
| 3 | Water Spray Cleaning | Non-potable water source | Biological / Chemical | PRP | -Evidence of tests conducted on water sources, to proof its potability -Programme for -Prerequisite Programme (PRP) – Cleaning practices |
|  |  | Proper cleaning of brushes | Biological | PRP |  |
|  |  | Re-contamination of dirty re-cycled water. | Biological | PRP |  |
| 4 | Pre-Sorting | Poor sorting standards/Damage of fruit | Q |  | -Handling procedure: Sorting -Prerequisite Programm(PRP) - Personnel Hygiene and Operating Standards, eg. handwash etc. -Prerequisite Programme (PRP) – Cleaning practices |
|  |  | Poor personnel hygiene standards | Biological | PRP |  |
|  |  | Poor cleaning standards of sorting conveyors/tables | Biological / Chemical | PRP |  |
| 5 | Hot water bath | Contact time | Q |  | -Prerequisite Programme (PRP) – Cleaning practices -Handling procedure: Hot water bath -Municipal water certificates -Laboratory results ensuring compliance to potable water standards -Procedure for bath handling-adequate filtering&bath change. |
|  |  | Temperature ( Some varieties such as Heidi must not exceed 30C, others operate at 50C) | Q |  |  |
|  |  | Non-potable water sources | Biological / Chemical | PRP |  |
|  |  | Insufficient cleaning of bath | Biological | PRP |  |
|  |  | Re-contamination of dirty re-cycled water. | Biological | PRP |  |
| 6 | OMEGA spray tunnel | Chemical contamination | Chemical | CCP | Procedure for controlling: -Sign-off sheets for chemical measurement. This might take the form of a batch sign off, and countersigned by authorized person. -Technical Responsility for determining suggested application quantities. -Standard for topping up of depleted/used chemicals, to ensure that active ingredient is always present in recommended volumes. |
|  |  | Re-contamination of fruit with dirty recycled water | Biological | CCP | -Procedure for bath handling-adequate filtering&bath change. |
|  |  | Contact time / temp | Q |  |  |
| 7 | Drying Tunnel (Pre-waxing) | Adequate drying of fruit to ensure effective waxing / to prevent fungal development in cartons | Q |  | Procedure: Drying tunnel operation |
|  |  | Continuous brushing may cause lenticel damage | Q |  |  |
| 8 | Waxing | Time / Temp-adequate waxing | Q |  | Procedure: Waxing unit operation |
|  |  | Chemical contamination-chemicals added to wax (overdosage) | Chemical | CCP | Procedure for controlling: -Sign-off sheets for chemical measurement. This might take the form of a batch sign off, and countersigned by authorized person. -Technical Responsility for determining suggested application quantities. -Standard for topping up of depleted/used chemicals, to ensure that active ingredient is always present in recommended volumes. |
| 9 | Drying Tunnel (Post-waxing) | Adequate drying of fruit to prevent fungal development in cartons | Q |  | Procedure: Drying tunnel operation |
|  |  | Continuous brushing may cause lenticel damage | Q |  |  |
| 10 | Sorting | Micro & Physical contamination | S (Biological / Chemical) | PRP | Prerequisite Programm(PRP) - Personnel Hygiene and Operating Standards, eg. handwash etc. |
|  |  | Handling | Q |  | -BOP Sorting -Inline QC Check – Sorting Effectiveness |
|  |  | Poor sorting | Q |  |  |
| 11 | Packing | Poor sizing | Q |  | BOP Sorting |
|  |  | Poor colour sorting | Q |  |  |
|  |  | Mechanical damage | Q |  |  |
|  |  | Handling | Q |  |  |
|  |  | Micro and Physical contamination | S (Biological / Chemical) | PRP | -Prerequisite Programm(PRP) - Personnel Hygiene and Operating Standards, eg. handwash etc. -Prerequisite Programme (PRP) – Cleaning practices -Prerequisite Programme (PRP) – Pest control programme |
|  |  | Scale calibration (inaccuracy) | Q | PRP | Yearly calibration of scales, and regular inhouse-checks. Records and certificates must be available. |
| 12 | Marking of Cartons | Incorrect number of ct / pallet | Q |  | BOP Carton Marking |
|  |  | Using wrong securing sheets | Q |  |  |
| 13 | Palletise | Wrong pallets used | Q |  | BOP Palletizing |
|  |  | Poor strapping practice | Q |  |  |
| 14 | Cooling / Container loading | Maintaining Temp (Q) Loading time | Q |  | -BOP Despatch -PPECB Container inspection / Temperature Inspection |
|  |  | Loading correct fruit to consignment order | Q |  |  |
|  |  | Container conditions | S (Biological /Physical) | PRP |  |

Typical CCP Control Plan – Mango Packing

| PROCESS STEP | HAZARD TYPE | PREVENTIVE MEASURE | CRITICAL LIMIT / MEASURE | MONITORING |  |  | RESP | CORRECTIVE ACTION | VERIFICATION |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | PROC | FREQ | REC. |  |  | METHOD | FREQ & RESP |
| Product Receiving | Chemical | -Adhere to product application standard -spraying programmes | Importing country MRL | Check withholding periods | Every batch | Spraying records | Packhouse Manager | Isolate and recall the product | Laboratory testing method | Every variety per season Packhouse Manager |
| Long Stem Bath | Chemical | Adherence to chemical application standard | Correct mixture/concentration | Check amount of chemical added to bath | Each batch | Batch make-up records | Packhouse Manager | Isolate and recall the product | Batch make up counter-checked by 2nd person | Every bath |
|  | Biological | Cleaning procedure for change of bath water | Amount of fruit washed | Monitor the amount of fruit through the bath | Amount of fruit through bath |  | Packhouse Manager | Isolate and recall the product | Do swabbing of fruit surface | Once per season to set bath operating standard |
| Omega Spray Tunnel | Chemical | Adherence to chemical application standard | Correct mixture / concentration | Check amount of chemical added to water |  | Batch make-up records | Packhouse Manager | Isolate and recall the product | Batch make up counter-checked by 2nd person | Every Batch |
|  | Biological | Cleaning procedure for change of bath water | Weight of fruit washed | Monitor the amount of fruit through the bath. | Amount of fruit through bath |  | Packhouse Manager | Isolate and recall the product | Do swabbing of fruit surface | Once per season to set bath operating standard |
| Waxing | Chemical | Adherence to chemical application standard | Correct mixture/concentration | Check amount of chemical added to wax mixture |  | Batch make-up records | Packhouse Manager | Isolate and recall the product | Batch make up counter-checked by 2nd person | Every Batch |

NB:  A Control Point is a CCP if the hazard cannot be controlled in a later step in the process, and there is no PRP (Prerequisite Programme) to bring it into control.