| 1 | Product Receiving | Truck standing temperature / time before off-loading | Q |  |  | Handling procedure |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Product standing – temperature / time after off-loading | Q |  |  | Handling procedure |
|  |  | Withoulding periods have been complied to | S (Chemical) | CCP |  | -Procedure for checking spraying records to allowable MRL. -Verification via laboratory test results. |
| 2 | Drenching | * Only applicable for some varieties(DPS). |  |  |  |  |
|  |  | Chemical – overdosing | S (Chemical) | CCP | Procedure for controlling: -Sign-off sheets for chemical measurement. This might take the form of a batch sign off, and countersigned by authorized person. -Technical Responsiility for determining suggested application quantities. -Standard for topping up depleted drench chemicals. -Training records for persons applying&handling chemicals. |  |
|  |  | Chemical - underdosing | Q |  |  |  |
|  |  | Changing of dirty drench chemicals | S (Biological) | CCP |  |  |
| 3 | Pre Cooling | Temperature over / under | Q |  | -Procedure for monitoring temperature (pulp and room). -Effective record-keeping. |  |
|  |  | Time over / under | Q |  |  |  |
|  |  | Handling of fruit | Q |  |  |  |
|  |  | Broken pallets / bins segregated | S(Physical) | PRP | -Procedure for minimizing contamination of wood, eg. a control procedure to illiminate chipped pallets/bins. |  |
|  |  | Light coverings | S(Physical) | PRP | -A control procedure to cover /laminate all lights and glass windows. -A checklist for monitoring glass breakage etc. |  |
| 4 | Presorting | General water quality | S (Chemical /Biological) | PRP | -Municipal water certificates -Laboratory results ensuring compliance to potable water standards |  |
|  |  | Poort sorting | Q |  | Inline QC Checks |  |
| 5 | Washing | General water quality | S (Chemical /Biological) | PRP | -Municipal water certificates -Laboratory results ensuring compliance to potable water standards |  |
|  |  | Concentration of Chemical(s) too high | S (Chemical) | CCP | Procedure for controlling: -Sign-off sheets for chemical measurement. This might take the form of a batch sign off, and countersigned by authorized person. -Technical Responsiility for determining suggested application quantities. -Standard for topping up of depleted/used chemicals, to ensure that active ingredient is always present in recommended volumes.- |  |
|  |  | Concentration of Chemical(s) too low | Q |  |  |  |
| 6 | Drying / Brushing | Overbrushing | Q |  | -Procedure for good operational practices should be in place. |  |
|  |  | Incomplete drying | Q |  | -Municipal water certificates -Laboratory results ensuring compliance to potable water standards. |  |
|  |  | General Water Quality | S (Chemical /Biological) | PRP |  |  |
| *7 | Waxing | * Only done in some cases |  |  |  |  |
|  |  | Under application | Q |  |  |  |
|  |  | Over application | Q |  | Procedure for controlling: -Sign-off sheets for chemical measurement. This might take the form of a batch sign off, and countersigned by authorized person. -Technical Responsility for determining suggested application quantities. |  |
|  |  | Under dosing chemical(s) | Q |  |  |  |
|  |  | Over dosing chemical(s) | S (Chemical) | CCP | BOP Drying after waxing |  |
|  |  | Mechanical damage | Q |  |  |  |
| *8 | Drying | Temp | Q |  | Prerequisite Programm(PRP) - Personnel Hygiene and Operating Standards, eg. handwash etc. |  |
|  |  | Time | Q |  |  |  |
|  |  | Micro & Physical contamination | S (Biological / Chemical) | PRP |  |  |
| 9 | Sorting | Handling | Q |  | -BOP Sorting -Inline QC Check – Sorting Effectiveness |  |
|  |  | Poor sorting | Q |  | BOP Sorting |  |
|  |  | Poor sizing | Q |  |  |  |
| 10 | Packing | Poor colour sorting | Q |  | -Prerequisite Programm(PRP) - Personnel Hygiene and Operating Standards, eg. handwash etc. -Prerequisite Programme (PRP) – Cleaning practices -Prerequisite Programme (PRP) – Pest control programme |  |
|  |  | Mechanical damage | Q |  |  |  |
|  |  | Handling | Q |  |  |  |
|  |  | Micro and Physical contamination | S (Biological / Chemical) | PRP |  |  |
|  |  | Scale calibration (inaccuracy) | Q | PRP | Yearly calibration of scales, and regular inhouse-checks. Records and certificates must be available. |  |
|  |  | Incorrect number of ct / pallet | Q |  | BOP Carton Marking |  |
| 11 | Marking of Cartons | Using wrong securing sheets | Q |  | BOP Palletizing |  |
|  |  | Wrong pallets used | Q |  |  |  |
| 12 | Palletise | Poor strapping practice | Q |  | -BOP Despatch -PPECB Container inspection / Temperature Inspection |  |
|  |  | Maintaining Temp (Q) Loading time | Q |  |  |  |
| 13 | Cooling / Container loading | Loading correct fruit to consignment order | Q |  |  |  |
|  |  | Container conditions | S (Biological /Physical) |  |  |  |

Typical CCP Control Plan – Topfruit Packing

| PROCESS STEP | HAZARD TYPE | PREVENTIVE MEASURE | CRITICAL LIMIT / MEASURE | MONITORING |  |  | RESP | CORRECTIVE ACTION | VERIFICATION |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | PROC | FREQ | REC. |  |  | METHOD | FREQ & RESP |
| Product Receiving | Chemical | -Adhere to product application standard -spraying programmes | Importing country MRL | Check withholding periods | Every batch | Spraying records | Packhouse Manager | Isolate and recall the product | Laboratory testing method | Every variety per season Packhouse Manager |
| *Drenching | Chemical | Adherence to chemical application standard | Correct mixture/concentration | Check amount of chemical added to bath | Each batch | Batch make-up records | Packhouse Manager | Isolate and recall the product | Batch make up counter-checked by 2nd person | Every bath |
|  | Biological | Cleaning procedure for change of drench water | Amount of bins drenched | Monitor the amount of bins through the bath | Bin count | Water change after bon count has been reached | Packhouse Manager | Isolate and recall the product | Do swabbing of fruit surface | Once per season to set bath operating standard |
| Washing | Chemical | Adherence to chemical application standard | Correct mixture / concentration | Check amount of chemical added to bath | Every bath mixture | Batch make-up records | Packhouse Manager | Isolate and recall the product | Batch make up counter-checked by 2nd person | Every bath |
| *Drying / Brushing | Chemical | Adherence to chemical application standard | Correct mixture / concentration | Check amount of chemical added to tank | Every bath mixture | Batch make-up records | Packhouse Manager | Isolate and recall the product | Batch make up counter-checked by 2nd person | Every bath |
| *Waxing | Chemical | Adherence to chemical application standard | Correct mixture/concentration | Check amount of chemical added to wax mixture | Every batch | Batch make-up records | Packhouse Manager | Isolate and recall the product | Batch make up counter-checked by 2nd person | Every Batch |

- Not applicable to all packhouses