| 1 | Receiving | -Presence of pathogens due to bad handling practices by pickers and during transportation. | Biological | PRP |  | -Supplier Quality Assurance -Approved Producer specifications -Receiving QC TO check compliance to specifications -Purchase from approved suppliers list |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | -Presence of pesticide residues on received product. | Chemical | CCP |  | -Supplier Quality Assurance -Approved Producer specifications -Receiving QC TO check compliance to specifications -Purchase from approved suppliers list -Verificaton of residues and witholding periods&residues of each product/season. |
|  |  | -Presence of foreign Objects e.g. woodsplinters, stones, insects, glass etc. | Physical | PRP |  | -Supplier Quality Assurance -Approved Producer specifications -Receiving QC TO check compliance to specifications -Purchase from approved suppliers list -Visual checks downstream, i.e washing |
| 2 | Storage / Weighing | -Introduction of pathogenic org. due to poor pest control. | Biological | PRP |  | -Cleaning programmes before the start of the season. -Pest Control Programme in place, with records of monitoring, sightings logs, and application of chemicals. |
|  |  | -Growth of pathogenic organisms due to poor stock rotation. | Biological | PRP |  | -FIFO -Adherance to product storage protocols (Time,Temp, Humidity) |
| 3 | Tipping | -Introduction of pathogenic organisms due to ineffective cleaning of equipment. | Biological | PRP |  | -Effective cleaning programme in place. Schedules and Records must be available. This must include Method, frequency,responsibility, chemicals used. |
|  |  | -Contamination of wood pieces, sand dirt etc. | Physical | PRP |  | -Procedure in place for washing, and detecting foreign bodies later in the process. |
| 4&6 | Conveyor | -Introduction of pathogenic organisms due to ineffective cleaning of equipment. | Biological | PRP |  | -Effective cleaning programme in place. Schedules and Records must be available. This must include Method, frequency,responsibility, chemicals used. -Only food grade cleaning chemicals may be used. |
|  |  | -Introduction of chemical hazard, due to ineffective cleaning-residues. | Chemical | PRP |  |  |
| 5 | *Washing | -Introduction of hazards due to water not being of potable standard | Chemical / Biological | PRP |  | -Procedure in place for monitoring that water complies to SABS 241:2001 |
|  |  | -Underdosing of sanitizing chemical | Biological | PRP |  | Procedure for controlling: -Sign-off sheets for chemical measurement. This might take the form of a batch sign off, and countersigned by authorized person. -Technical Responsiility for determining suggested application quantities. -Standard for topping up of depleted/used chemicals, to ensure that active ingredient is always present in recommended closes. |
|  |  | -Overdosing of sanitizing chemicals | Chemical | CCP |  |  |
|  |  | -Introduction of physical hazards such as sand, insects, due to filtering system not filtering sufficiently. | Physical | PRP |  | -Equipment maintenance programme must be in place, with records of maintenance work done. -Cleaning programme in place with records of filter cleaned. |
| 7 | Hand sorting | -Introduction of hazards due to ineffective cleaning of sorting surfaces. | Biological/Chemical | PRP |  | -Effective cleaning programme in place. Schedules and Records must be available. This must include Method, frequency,responsibility, chemicals used. -Only food grade cleaning chemicals may be used. |
|  |  | -Introduction of hazards due to bad personnel hygiene practices | Biological / Physical | PRP |  | -Personnel hygiene policy must be in place with handwash procedure, jewellery policy, plaster policy etc. -Training records for staff handling produce, in personnel hygiene practices. |
| 8 | Size Grading | -Introduction of hazards due to ineffective cleaning of sorting surfaces. | Biological/Chemical | PRP |  | -Effective cleaning programme in place. Schedules and Records must be available. This must include Method, frequency,responsibility, chemicals used. -Only food grade cleaning chemicals may be used. |
|  |  | -Introduction of hazards due to bad personnel hygiene practices | Biological / Physical | PRP |  | -Personnel hygiene policy must be in place with handwash procedure, jewellery policy, plaster policy etc. -Training records for staff handling produce, in personnel hygiene practices. |
| 9 | Packing | -Introduction of hazards due to bad personnel hygiene practices | Biological / Physical | PRP |  | -Personnel hygiene policy must be in place with handwash procedure, jewellery policy, plaster policy etc. -Training records for staff handling produce, in personnel hygiene practices. |
|  |  | -Introduction of pathogens due to poor storage practices of packaging material | Physical/Biological | PRP |  | -All storage areas must be included in the Pest Control and Cleaning Programmes. Records must be kept. -Precautions must be taken to prevent contamination of dust and other contaminants. -FIFO |
| 10&11 | Weigh/Seal | -Introduction of hazards due to bad personnel hygiene practices | Physical/Biological | PRP |  | -Personnel hygiene policy must be in place with handwash procedure, jewellery policy, plaster policy etc. -Training records for staff handling produce, in personnel hygiene practices. |
| 12 | Labelling | No hazards |  |  |  |  |
| 13 | Cold Storage | -Introduction of pathogenic org. due to poor pest control. | Biological | PRP | -Pest Control Programme in place, with records of monitoring, sightings logs, and application of chemicals. |  |
|  |  | -Growth of pathogenic organisms due to poor stock rotation. | Biological | PRP | -FIFO -Adherance to product storage protocols (Time,Temp, Humidity) |  |
| 14 | *Metal Detection | -Ineffective working of metal detector | Physical | CCP | -Maintenance records -Calibration records |  |
| 15 | Dispatch | No hazards |  |  |  |  |

*  Only applicable to certain operations

Typical CCP Control Plan – Vegetable Packing

| PROCESS STEP | HAZARD TYPE | PREVENTIVE MEASURE | CRITICAL LIMIT / MEASURE | MONITORING |  |  | RESP | CORRECTIVE ACTION | VERIFICATION |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | PROC | FREQ | REC. |  |  | METHOD | FREQ & RESP |
| Product Receiving | Chemical | -Adhere to product application standard -spraying programmes | Importing country MRL | Check withholding periods | Every batch | Spraying records | Packhouse Manager | Isolate and recall the product | Laboratory testing method | Every variety per season Packhouse Manager |
| *Metal Detection | Physical | Good Supplier foreign body control | None | Receiving QC checks | Every delivery | Receiving QC checks | Receiving QC | Reject delivery |  |  |